AccessionIndex: TCD-SCSS-T.20250922.001
Accession Date: 22-Sep-2025
Accession By: Dr.Brian Coghlan
Object name: Intel C3000 bit-slice chipset and associated chips
Vintage: 1974
Synopsis: Intel’s bipolar microprogrammed bit-slice processor.
Description:
Intel’s C3000 [1][2][3][4][5] was a Schottky bipolar bit-slice chipset introduced in September, 1974. It consisted of the C3001 microprogram control unit and the C3002 central processing element, both intended for use in microprogrammed computer architectures.
Microprogramming was invented by Maurice Wilkes at Cambridge University, U.K. in 1951 [6]. Even by then a computer was understood to consist of a central processing unit (CPU), memory and input/output. Wilkes proposed that the CPU hardware would be simpler if a very basic CPU could be microprogrammed to emulate a higher-level instruction set of the user-level CPU and thereby execute programs in the user-level memory (Fig.1). This simple CPU would be composed of a microprogram control unit, microprogram memory and a central processing element.
This concept was of great value while hardware (both logic and memory) was expensive, as it was for the vacuum-tube era of the early 1950s and the transistor era of the late 1950s and early 1960s, as well as for the early microprocessor era of the 1970s and 1980s. Since then the ability to fabricate billions of inexpensive transistors on a chip has rendered this a niche value concept.
Intel designed the C3000 chipset in the early 1970s to implement this concept. The microprogrammed CPU would comprise a C3001 microprogram control unit (MCU), microprogram memory and multiple C3002 central processing elements (CPEs), see Figs.2-3. The MCU would address its own microprogram memory, with some of its contents being used to control the program sequence. But other of the contents would be distributed to the CPEs to control their operations. Fig.4 shows a C3001 MCU and 16-bit C3002 array (non-pipelined) configuration. Adding a pipeline register as in Fig.5 would speed up execution. Vectored interrupts could be added using the C3214 interrupt control unit, see Fig.6.
The C3001 included the microprogram address register, selected the next micro-instruction as needed. Its architecture is shown in Fig.7. It could address 512 microinstructions using a 9-bit microprogram address register, with provision for selection of the next microinstruction based on the contents of the microprogram address register, or a jump function, or decoded and tested data from several busses, or microprogram interrupts, with 11 address control functions selected by the next address logic, including 3 jump & test latch functions, and a 16-way jump & test instruction bus function. It had 2 flag registers, with 8 flag control functions. The sequencer was difficult to write code for due to the variety of row and column jumps to get from point A to point B in the address map.
The C3002 central processing element (CPE) was a 2-bit arithmetic logic unit (ALU) plus a register file. Its architecture is shown in Fig.8. Multiple of these could be connected to build a wider word-length ALU. It had 3 input data busses and 2 tristate output data busses, 11 general purpose registers, an accumulator, and an independent memory address register. It was able to execute any of over 40 functions in 8 function groups, e.g. 2’s complement arithmetic, logical AND, OR, NOT and exclusive-OR, increment/decrement, shift left/right, bit test and zero detection, with internal carry look-ahead generation. Faster external carry/borrow propagation could be added using the C3003 carry look-ahead adder, see Fig.9. Its main drawback was that it was only a 2-bit slice, whereas competitors were mostly 4-bits, and eventually 8-bits and even later 16-bits.
In addition Intel provided the following support chips, see [7]:

	Part
	Function

	3003 
	Carry-lookahead adder

	3205 
	1 of 8 binary decoder

	3207 
	Quad Bipolar-to-MDS Level Shifter and Driver

	3208 
	Hex Sense Amplifier for MOS Memories

	3210 
	TTL-to-MOS Level Shifter and High Voltage Clock Driver

	3211 
	ECL-to-MOS Level Shifter and High Voltage Clock Driver

	3212 
	Multimode latch buffer

	3214 
	Priority interrupt unit

	3216 
	Noninverting bidirectional bus driver

	3222 
	Refresh Controller for 4K NMOS DRAMs

	3226 
	Inverting bidirectional bus driver

	3232 
	Address Multiplexer and Refresh Counter for 4K DRAMs

	3235 
	Quad Bipolar-to-MOS Level Shifter and Drive

	3242 
	Address Multiplexer and Refresh Counter for 16K DRAMs

	3245 
	Quad Bipolar TTL-to-MOS Level Shifter and Driver for 4K

	3246 
	Quad Bipolar ECL-to-MOS Level Shifter and Driver for 4K

	3404 
	High-performance 6-bit Latch

	3408 
	Hex Sense Amp and Latch for MOS Memories

	3601 
	256x4-bit programable read-only memory

	3604 
	512x8·bit programable read-only memory

	3301A 
	256x4-bit read·only memory

	3304A 
	512x8-bit read-only memory


The chipset was DTL and TTL compatible with a 5V±5% power supply, dissipated 0.85-1.26W, had a 0°C-70°C operating temperature range, could be clocked at up to 10MHz, and had a minimum cycle time of 85nS. The C3000 was second sourced by Signetics and Intersil. Notably, Signetics then designed and manufactured an 8080 emulator based on the C3000 chipset [8].
Many thanks to Brian Coghlan for donating these items.
The homepage for this catalog is at: https://www.scss.tcd.ie/SCSSTreasuresCatalog/
Click 'Accession Index' (1st column listed) for related folder, or 'About' for further guidance.

Some of the items below may be more properly part of other categories of this catalog, but are listed here for convenience.

	Accession Index
	Object with Identification

	TCD-SCSS-T.20250922.001


	Intel C3000 bit-slice chipset and associated chips. Intel’s bipolar microcoded bit-slice processor. 1974.

	TCD-SCSS-T.20250922.001.01


	1 x Intel C3001 microprogram control unit.

	TCD-SCSS-T.20250922.001.02


	13 x Signetics N3002 central processing element.

	TCD-SCSS-T.20250922.001.03


	1 x Intel C3003 carry look-ahead adder.

	TCD-SCSS-T.20250922.001.04


	1 x Intel C3214 priority interrupt unit.

	TCD-SCSS-T.20250922.001.05


	1 x Intel B3222 DRAM controller.

	TCD-SCSS-T.20250922.001.06


	1 x Intel D3242 address multiplexer and refresh controller.

	TCD-SCSS-X.20250916.001


	Dr.Brian Coghlan's Collection of Early Microprocessors. An extensive and nearly complete set of unused 1970s microprocessor chips, most accompanied with documentation, some with demonstration boards. 1971.

	
	


References:
1. Intel, Intel 3001 Microprogram Control Unit, see:

https://www.scss.tcd.ie/SCSSTreasuresCatalog/hardware/TCD-SCSS-T.20250922.001/Intel-3001-datasheet.pdf
Last browsed to on 22-Sep-2025.

2. Intel, Intel 3002 Central Processing Element, see:

https://www.scss.tcd.ie/SCSSTreasuresCatalog/hardware/TCD-SCSS-T.20250922.001/Intel-3002-datasheet.pdf
Last browsed to on 22-Sep-2025.

3. Intel, Intel 3003 Look-ahead Carry Generator, see:

https://www.scss.tcd.ie/SCSSTreasuresCatalog/hardware/TCD-SCSS-T.20250922.001/Intel-3003-datasheet.pdf
Last browsed to on 22-Sep-2025.

4. Intel, Intel 3014 Interrupt Control Unit, see:

https://www.scss.tcd.ie/SCSSTreasuresCatalog/hardware/TCD-SCSS-T.20250922.001/Intel-3014-datasheet.pdf
Last browsed to on 22-Sep-2025.

5. CPUshack, The Collector’s Guide to Vintage Intel Microchips, see:

https://www.cpushack.com/wp-content/uploads/2018/06/VintageIntelMicrochipsRev4.pdf
Last browsed to on 22-Sep-2025.

6. Maurice Wilkes, The Best Way to Design an Automatic Calculating Machine, In: Manchester University Computer, Inaugural Conference, July 1951, pp.16-18; reprinted in Annals of the History of Computing, pp.118-121, Vol.8, 1986, and in The Early British Computer Conferences, M.R. Williams and M. Campbell-Kelly, eds., Charles Babbage Institute Reprint Series for the History of Computing, pp.182-184, Vol.14, MIT Press, Cambridge, Mass., and Tomash Publishers, Los Angeles, 1989.
7. Intel, Intel C3000 Datasheets, see:

https://www.scss.tcd.ie/SCSSTreasuresCatalog/hardware/TCD-SCSS-T.20250922.001/Intel-C3000-datasheets/
Last browsed to on 22-Sep-2025.

8. Signetics, Signetics 8080 Emulator Manual, March 1977, see:

https://bitsavers.org/components/signetics/Signetics_8080_Emulator_Manual_Mar77.pdf
Also: https://www.scss.tcd.ie/SCSSTreasuresCatalog/hardware/TCD-SCSS-T.20250922.001/Signetics-8080EmulatorManual-Mar1977.pdf
Last browsed to on 22-Sep-2025.

[image: image1.jpg]EXTERNAL CONTROL MAIN MEMORY
SIGNALS ADDRESS BUS

DATA
CONTROL PROCESSING
MEMORY MICRO-INSTRUCTION) UNIT

MICROPROGRAMS)) (ALU, REGISTERS,

BUS CONNECTIONS)

MAIN MEMORY
(MACROPROGRAMS))

EXECUTION
CONTROL.
UNIT

MACRO-INSTRUCTION MACRO-INSTRUCTIONS/DATA
OPCODE H

S SIS d

CENTRAL PROCESSING UNIT





Figure 1: Microprogrammed central processing unit and user-level memory.

[image: image2.jpg]conrrol 7o
wrERORY AND 10

CrE FONETION
575

3002
CPE ARRAY

eI
SROBE
27




Figure 2: Intel marketing graphic of C3000 central processing unit.

[image: image3.jpg]RO TAUCH D!

FLAG OUTPUT
FUNCTION BUS

MICRO.PROGRAM MICRO.PROGRAM Mcu
MEMORY MEMORY ADDRESS c301




Figure 3: Intel C3000 central processing unit (non-pipelined).

[image: image4.jpg]controL 1o MEMORY ADDRESS  DATA BUS TO
MORY 1/0 BUs MEMORY.

CPARRAY.
(18US NOT SHOWN)





Figure 4: Intel C3001 and 16-bit C3002 array (non-pipelined).

[image: image5.jpg]conTROL TO MEMORY ADDRESS ~ DATA BUS TO
MEMORY 1/0. 8US MEMORY

P ARRAY
(18US NOT SHOWN)





Figure 5: Pipelined Intel C3001 and 16-bit C3002 array.

[image: image6.jpg]



Figure 6: Adding the C3214 interrupt control unit to the Intel C3001 microprogram control unit.

The C3214 provides vectored interrupts.


[image: image7]
Figure 7: Intel C3001 microprogram control unit (MCU) architecture.


[image: image8]
Figure 8: Intel C3002 central processing element (CPE) architecture.

[image: image9.jpg]+5v

[ e | ”

E — [(ec,+s

3003 c C+8
X1 ¥y Cat1 Xo¥o X1¥s Cat?  Xg¥s gani¥s Cath

3003 Ca

Xo Yo

XY
CARRY 3002 3002
FROM
3001
CARRY
T0

3001




Figure 9: Adding the C3003 carry-lookahead adder to the Intel C3002 central processing element.

[image: image10.jpg]1n0 w01y OY

vivo3levns 03

jsnax| |sne
oxtul, Jo 4y
i
50y
swasiosy ¥300930
Gvaroivuzs Sh
_ NoMBRNS
) v
T st
(0w 1m0
0TI INHLIEY
i # L
wasoan
wasioamov Sauoow
iowaw
s sz
inaino insino
|+ |
o 1y
) Shay

NOILONN 360

llT. V3 $S3HOQV 318VNI




[image: image11.jpg]sna sng

s_zgtaé.z,__g_azz
snoxs I kA T
X 38 0 H\ 8
ol [BE] fos & gg
r—— e = ) 7. S S \ iy
| + Worvia |
e ==
B Meziime -
| - atne] [sovszo
oo s T
Woivy tua ]| 4a4dne [—
wegbous boa 1
[E———
Lo
| LT TTTTT
o L+
sassiots awcay
s -
| o
Do sn8
| €Y NOILONNA
R ) v "
}
w8 o i
| TTos wow o
it naino]  [usasneinasno
L oww T o " g PR —
e e ] S S——— ]
SNB SS3HAQY wHi Ldnyy3ing
AvonEn w8 ouom seyutay




