.pl 62

Chapter 1

Introduction and Overview

#1.1 INTRODUCTION#

The  DSI-32  Assembler  enables  the  user  to  assemble  NS32000

assembly  language statements.  The Assembler produces an  object

file  that may then be linked by the DSI-32 Linker to produce  an

executable program.  It is recommended that the user have a  copy

of  the  National  Semiconductor Instruction Set manual  for  the

32000 series of microprocessors. (Order from Definicon Systems or

National Semiconductors: Document NSP-INST-REF-M).

The   manual  contains  a  detailed  description  of  the  DSI-32

Assembler  language.  The  various chapters  that  comprise  this

manual contain the following information:

- Chapter  1,  Introduction and Overview.  Introduces the DSI-

32 and lists its features.

- Chapter  2,  DSI-32 Language Elements.  Defines the form  of

DSI-32 Assembler statements,  constants,  terms, symbols and

expression syntax.

- Chapter  3,  DSI-32  Programs.  Describes  the  support  for

modules,  segments,  relocation modes,  the various location

counters and the various assembly modes.

- Chapter  4,  Instruction  Operands.  The  syntax  of  DSI-32

operands is described.

- Chapter 5,  DSI-32 Assembler Pseudo-Operands.  The syntax of

the DSI-32 pseudo-ops is described.

- Chapter 6, Procedures. Defines the format of a procedure and

the syntax of procedure control pseudo-ops.

- Chapter  7,  Invocation and Operation.  The operation of the

Assembly is described as well as options and error messages.

- Appendix  A,  Reserved Symbols.  The DSI-32 reserved symbols

are defined.

.pa

#1.2  OVERVIEW OF THE DSI-32 ASSEMBLER#

The  DSI-32  Assembler provides a variety of features  which  aid

assembly language programming.

- Assembler Input  and  Output  Files.

An object code file and an optional listing file are produced  by

the  DSI-32  Assembler.  The object code file once linked by  the

DSI-32  Linker forms an executable file which may then be  loaded

by  the  Definicon  Systems' LOAD program.  The listing  file  is

formed  from  the assembled code,  the source  line  number,  the

source statements and any error messages. Input and Output files,

listing file format,  and error messages are described in Chapter

7.

- Instruction  Set.

The  complete NS32032 instruction set is supported by the  DSI-32

Assembler.    All   the  memory-management   and   floating-point

instructions are also supported by the Assembler.

- Addressing  Modes.

The Assembler supports the nine general addressing modes:

(1)  register

(2)  register relative

(3)  memory

(4)  immediate

(5)  absolute

(6)  top of stack

(7)  external

(8)  memory relative

(9)  scaled index

- Data  Types.

The  Assembler supports the following data  types:  byte,  signed

byte,  word,  signed  word and  double-word,  signed  double-word

integers  single  and  double-precision  floating-point  numbers,

packed decimal numbers, bits, and bit fields.

- Assembler Pseudo Opcodes.

The Assembler provides a variety of pseudo-ops to define symbols,

allocate  storage,  allocate  constants,  manage  procedures  and

control program segments.

- Procedures.

A  special  set of pseudo-ops,  called the  procedure  management

pseudo-ops,  are  supported by the assembler to make the creation

of  program  procedures  simple.   These  pseudo-ops  enable  the

programmer  to  write modular routines that may recursively  call

themselves.  The pseudo-ops create a frame on stack for temporary

data  storage  of  the  parameters,   return  values,  and  local

variables of the procedure.

#1.3  NS32000 REGISTERS#

The  NS32000 system consists of four sets of register types which

are detailed below:

- General-Purpose registers (on the CPU)

- Dedicated registers (on the CPU)

- Floating-Point registers (on the FPU)

- Memory-Management registers (on the MMU)

#1.3.1  General-Purpose Registers#

The eight General-Purpose registers are:  R0, R1, R2, R3, R4, R5,

R6 and R7.

Each  general purpose register is 32 bits long and may be used in

byte,  word,  and  double word operations.  Only the bits in  the

length specified by the instruction type are affected.

General  purpose  registers  may  be combined  to  form  even/odd

register pairs as follows:   R0/R1,  R2/R3,  R4/R5, R6/R7. The 64

bit  register  pairs  are used  by  various  extended  arithmetic

instructions which produce a 64 bit result.  Register pair  names

are: R0, R2, R4, and R6.

.pa

#1.3.2  Dedicated Registers#

The   eight  dedicated  registers  contain  pointers  to   memory

locations and status information.

Register                Name      Contents

--------                ----      --------

Program Counter          PC       pointer to current instruction

Interrupt Stack Pointer  SP0      pointer to top Interrupt Stack

User Stack Pointer       SP1      pointer to top of User Stack

Frame Pointer            FP       pointer to current Frame

Static Base              SB       pointer to current Static Base

Area

Interrupt Base           INTBASE  pointer  to Exception Dispatch

Table

Module                   MOD      pointer   to  current   Module

Descriptor

Processor Status         PSR      processor status flags

The  PC, SP0, SP1, FP, SB and INTBASE registers are 32 bits long.

However, since these registers contain memory addresses, and only

the  low-order  24 bits are used in the NS32032 the high-order  8

bits are zeroed by the processor, the assembler treats them as 32

bit registers.  The MOD and PSR registers are 16 bits  long.  The

MOD  register  contains a memory pointer,  and the  PSR  register

contains  the  processor  status flags.  Even  though  two  stack

pointers have been described a program uses only one of the stack

registers  at any given time.  The particular register in use  is

determined by the context of the processor.

The  CPU selects SP0 as the current stack pointer if the 'S'  bit

in the PSR is 0, and SP1 if the 'S' bit is 1. This enables 'user'

and 'supervisor' stacks to be separated.

The  currently selected stack is referenced via the SP symbol.  A

program may only change which stack will be accessed, by changing

the  'S' bit.  The Assembler does  not,  therefore,  support  the

symbols SP0 and SP1.

The  PSR  status  flags  define the current CPU  status  and  the

results  of  previous instruction(s).  The layout of the  PSR  is

shown below:

.pa

Supervisor Flags                   User Flags

+-------------------------------+-------------------------------+

| x   x   x   x | I | P | S | U | N | Z | F | x   x | L | T | C |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+

15                                                              0

x - reserved

The status flags have the following functions:

User Flags (available in both user and supervisor modes)

----------

C       is  the carry flag.  The 'C' bit indicates a  carry  or

borrow   occurred  after  an  addition  or  subtraction

instruction. 'C' is used in the calculation of multiple

precision arithmetic.  A value of '1' means a carry  or

borrow, a value of '0' means no carry or borrow.

T       is  the  Trace Trap flag.  The 'T' bit enables  program

tracing.  If  this bit is a '1' a TRC trap is  executed

after each instruction.  See also 'P' flag.

L       is  the  Low flag.  The 'L' bit is  set  by  comparison

instructions.   In a comparison instruction the 'L' bit

is  set to '1' if the second  operand is less than  the

first,  when  both operands are interpreted as unsigned

integers, otherwise it is set to '0'.

F       is the General Condition flag. The 'F' bit is a general

flag which is used by many instructions.

Z       is  the Zero flag.   The 'Z' flag is set by  comparison

instructions.   In a comparison instruction the 'Z' bit

is  set to a '1' if the second operand is equal to  the

first operand, otherwise it is set to '0'.

N       is the Negative flag. The 'N' flag is set by comparison

instructions.   In a comparison instruction the 'N' bit

is  set  to '1' if the second operand is less than  the

first  operand,  when both operands are interpreted  as

signed integers, otherwise it is set to '0'.

Supervisor Flags (available to supervisor program only)

----------------

U       is  the  user Mode flag.  The User Mode flag  sets  the

current mode of system operation.  If 'U' bit is set to

'1'   no  privileged  instructions  may   be   executed

otherwise            if 'U' is 0,  the system is in the

supervisor mode and all instructions may be executed.

.pa

S       is  the Stack flag.  The 'S' bit specifies whether  the

SP0  or  the SP1 register is to be used  as  the  stack

pointer.  The  'S'  bit  is  automatically  cleared  on

interrupts  and  traps.  It may have a setting  of  '0'

(SP0) or '1' (SP1).

P       is  the Trace Pending flag.  The 'P' bit prevents a TRC

trap from occurring more than once for an  instruction.

The 'T' flag contents are copied to the 'P' flag at the

start of an instruction. At the end of the instruction,

if  P is 1,  a trace trap is taken otherwise if P is 0,

no trace trap is taken.  The 'P' flag is cleared when a

trace  is  taken to prevent a second  trace  trap  from

occurring.

I       is the Interrupt flag. Interrupts are enabled If 'I' is

'1' otherwise if 'I' is '0', then all interrupts except

the Non-Maskable Interrupt are disabled.

.pa

#1.3.3  Floating-Point Registers#

The  Floating-Point  registers are primarily intended to  provide

fast  temporary  storage  for  floating  point  operations.   The

eight floating point registers are:  F0,  F1,  F2, F3, F4, F5, F6

and  F7.   The  registers are 32 bits long and may  hold  single-

precision ieee floating-point numbers.

The  Floating-Point  registers  may be combined to  form  64  bit

even/odd register pairs: F0/F1, F2/F3, F4/F5, F6/F7.  Each of the

register  pairs may hold a double-precision  ieee  floating-point

number.  The high order part of the floating point number is held

in  the   odd  register and the low order part  is  in  the  even

register.  Register pair names are: F0, F2, F4, and F6.

The  Floating-Point  Status Register (FSR) describes the  current

mode of the FPU as well as any error status due to the  execution

of the last floating point instruction.

#1.3.4  Memory-Management Registers#

The  Memory  Management  registers  enable  the  various  options

available  in  the  MMU to be manipulated.  The  various  Memory-

Management registers are described below.  All  Memory-Management

registers, except SC0 and SC1, are 32 bits long.  The SC0 and SC1

logical  registers are 16 bits long and occupy a single  physical

32 bit register within the MMU.

Register                     Name       Function

--------                     ----       --------

Page  Table  Register  0     PTB0       Specifies  the current

base address of the Level

1 Page Table.

Page Table Register 1        PTB1       Specifies the current

base address of the  user

mode Level  1 Page Table.

Error/Invalidate Address     EIA        When an error  occurs, it

contains   the    virtual

address that  caused  the

exception to occur.  When

the register  is  written

to it informs the MMU  to

remove the entry from the

cache in the MMU.

.pa

Breakpoint  Register  0      BPR0       Specifies  the address at

which a  breakpoint is to

occur.

Breakpoint Register 1        BPR1       Specifies the  address at

which a breakpoint  is to

occur.

Breakpoint Count             BCNT       Specifies the  number  of

breakpoints   that   have

occurred      at      the

BPR0

address.  Once the  count

reached zero then a break

will be generated.

Program Flow 0               PF0        Contains  the address  of

the  previous  nonsequen-

tial instruction.

Program Flow 1               PF1        Contains  the address  of

the   last  nonsequential

instruction.

Sequential Count 0           SC0        The count of instructions

executed  since the  last

nonsequential instruction

Sequential Count 1           SC1        The count  of  sequential

instructions     executed

prior  to   the last non-

sequential instruction

Memory Status Register       MSR        Contains  the status  and

mode of the MMU.

.pa

#1.4  DOCUMENTATION CONVENTIONS#

The  following documentation conventions are used  in  describing

the DSI-32 Assembler.

Upper-case  letters show the instruction and pseudo  op

mnemonics.   The  mnemonics must be entered exactly  as

shown.

Spaces or blanks,  when present,  are significant; they

must   be  entered  as  shown.    Multiple  blanks   or

horizontal tabs may be used in place of a single blank.

<> - angle  brackets  enclose  descriptive names  (in  lower

case) for user-supplied operands.

{} - braces enclose two or more items of which one, and only

one,  must be used.   The items are separated from each

other by a logical OR sign "|".

[] - brackets enclose optional item(s).

| - logical OR sign separates items of which one,  and only

one, may be used.

... - three consecutive periods indicate optional  repetition

of the preceding item.

,,, - three  consecutive commas indicate optional  repetition

of  the  preceding item.   Items must be  separated  by

commas.