.pl 62

CHAPTER 4

INSTRUCTION OPERANDS

#4.1  INTRODUCTION#

The  syntax  of  instruction  operands are  desribed  in  detail.

Instruction  operands  are operated on by CSD-32000  opcodes  and

psuedo-ops. In the following example the opcode MOVW means move a

16 bit integer. VAL1 and VAL2 are the instruction operands.  They

identify  the source and destination for the number to be  moved.

Instruction operands may be constants, memory addresses, symbols,

and/or  expressions.

Example  ----->    MOVW  VAL1,VAL2

The following operand types exist:

Operand Type                         Section

General Operands                               4.2

Register Operands                            4.2.1

Register Relative Operands                   4.2.2

Frame Memory Operands                        4.2.3

Frame Memory Relative Operands               4.2.4

Stack Memory Operands                        4.2.5

Stack Memory Relative Operands               4.2.6

Static Memory Operands                       4.2.7

Static Memory Relative Operands              4.2.8

Program Memory Operands                      4.2.9

Immediate Operands                           4.2.10

Absolute Operands                            4.2.11

External Operands                            4.2.12

Top of Stack Operands                        4.2.13

Scaled-Index Byte Operands                   4.2.14

Scaled-Index Word Operands                   4.2.15

Scaled-Index Double-Word Operands            4.2.16

Scaled-Index Quad-Word Operands              4.2.17

Immediate subrange operands                    4.3

Quick                                        4.3.1

Block Length                                 4.3.2

Bit Field Length                             4.3.3

Bit Field Offset                             4.3.4

Displacement Operands                        4.3.5

Program Memory Operands                        4.4

Register Operands                              4.5

Register List Operand                          4.6

Configuration List Operand                     4.7

Processor Register Operands                    4.8

Memory Management Register Operands            4.9

External Procedure Operands                    4.10

#4.2  General Operands#

Format:   <gen>

description:

<gen>          is   one   of  the   following   General

Addressing Operand types:

Register

Register Relative

Frame Memory

Frame Memory Relative

Stack Memory

Stack Memory Relative

Static Memory

Static Memory Relative

Program Memory

Immediate

Absolute

External

Top of Stack

Scaled-Index Byte

Scaled-Index Word

Scaled-Index Double-Word

Scaled-Index Quad-Word

Many operands use displacements (disp) to specify the offset from

a base address to a particular memory location.  The displacement

range depends on the particular 32000 series microprocessor  that

is in use.  The 32008,  32016 and 32032 currently have 24 address

externally  and thus the displacement must range from -2**24+1 to

2**24-1 (ie from -16777215 through to 16777215).  The newer 32000

family  microprocessors have more external lines,  eg  the  32332

carries  all out 32 address lines  and thus the displacement  can

be  any value in the entire 30 bit range ie -2**29+1  through  to

2**29+1.

Note.  The  assembler  computes all displacements to the full  30

bits and thus can be used with any 32000 series microprocessor.

Subsections  4.2.1 through 4.2.17 detail the syntax and  function

of each of the general operand types.

.pa

#4.2.1  Register Operands#

Format:   <register>

description:

<register>     is  a symbol,  either reserved or  user-

defined,   with  register  type.    (See

Section 2.6.3).

A  register  operand  specifies one  of  the  General-Purpose  or

Floating-Point registers.

Floating-Point  registers  are analogous to  the  General-Purpose

registers  except  they  should only be used  for  Floating-Point

instructions.

Example:

RG5:   .EQU  R5

FREG4: .EQU  F4

MULF  FREG4,F6

SUBW  RG5,R3

The  MULF  instruction  multipies a  floating-point  number  from

Floating-Point  register   F4  with Floating-Point  register  F6.

The SUBW instruction subtracts the low-order word of R5 from R3.

.pa

#4.2.2.  Register Relative Operands#

Format:   <disp>(<register>)

description:

<disp>         is  a  displacement or expression  which

must evaluate to an immediate type  with

a   value  in  the  range  -16777215  to

16777216.

(<register>)   is  a  symbol with  register  type  (see

Section    2.6.3).    Parentheses    are

required.

A  register  relative  operand specifies an operand at  a  memory

address.  The address is the sum of the displacement disp and the

contents of the General-Purpose register R<n>.

Examples:

1.  MOVW  VALUE, 3(R4)

2.  SUBB  (TWO*3)(R3), VALUE

In  example  1,   3(R4)  is  a  register  relative  operand.  The

instruction moves the word at the address specified by the symbol

VALUE to the memory location specified by 3(R4).

In  example 2,  (TWO*3)(R3) is a register relative  operand.  The

expression TWO * 3 must evaluate to an immediate type. This value

is  added  to the contents of register R3 to yield the  operand's

address.  The instruction subtracts the byte at this address from

the byte at the address specified by the symbol VALUE. The result

is stored at VALUE.

.pa

#4.2.3  Frame Memory Operands#

Format:  <disp>(FP)

description:

<disp>         is  a  displacement or expression  which

must evaluate to an Immediate type  with

a  value  that  is legal for  the  32000

series microprocessor in use.

(FP)           specifies  the  Frame-Pointer  register.

Parentheses are required.

A  frame memory operand specifies an operand at a memory address.

The address is the sum of the displacement disp and the  contents

of the FP register.

A  symbol  may be used as a frame-memory operand if it  has  been

defined as FP type (see Section 2.6.3).

Examples:

1.  MOVW  VALUE, 12(FP)

2.  SUBB  (TWO*3)(FP), VALUE

In example 1,  12(FP) is a frame memory operand.  The instruction

moves  the  word at the address specified by the symbol VALUE  to

the word at the memory address specified by 12(FP).

In  example  2,  (TWO*3)(FP)  is  a  frame  memory  operand.  The

expression  (TWO*3)  must evaluate to an  immediate  value.  This

value  is  added  to the contents of register  FP  to  yield  the

operand's  address.  The  instruction subtracts the byte at  this

address  from  the byte at the address specified  by  the  symbol

VALUE.  The result is stored at VALUE.

.pa

#4.2.4  Frame memory Relative Operands#

Format:   <disp>(<expression>)

description:

<disp>         is  a  displacement or expression  which

must evaluate to an Immediate type  with

a  value  that  is legal for  the  32000

series microprocessor in use.

(<expression>) is  an expression with FP relative  type

(see  Section 2.6.3).   Parentheses  are

required.

A  frame memory relative operand points to a memory address whose

double-word  value  is  added to the  displacement  to  give  the

resultant address.

Example:

FP_VAL:	.EQU	12(FP)

ADDB	R3,12(5(FP)+1)

MOVB	3(FP_VAL),R4

In  the  above  example 12(5(FP)+1) is a  frame  memory  relative

operand.   The  second assembler instruction adds register R3  to

the  specified  address.   The address is the sum of 12  and  the

double-word value at the address (5(FP+1)) ie (6(FP)). The symbol

FP_VAL  is  defined as FP relative in the  first  statement.  The

third  statement  moves a byte from the double-word at  offset  3

from FP_VAL to R4.

.pa

#4.2.5  Stack Memory Operands#

Format:   <disp>(SP)

description:

<disp>         is  a  displacement or expression  which

must evaluate to an Immediate type  with

a  value  that  is legal for  the  32000

series microprocessor in use.

(SP)           specifies  the  current  stack  pointer.

Parentheses are required.

A  stack memory operand specifies an operand at a memory address.

The  address is computed as the sum of the displacement disp  and

the contents of the current stack pointer register.

A  symbol  may be used as a stack memory operand if it  has  been

defined as an SP type (see Section 2.6.8).

Examples:

1.  MOVW  VALUE, 12(SP)

2.  SUBB  (TWO*3)(SP), VALUE

In example 1,  12(SP) is a stack memory operand.  The instruction

moves  the word at the address specified by the symbol  VALUE  to

the   memory  address  specified  by  12(SP).

In  example  2,  (TWO*3)(SP)  is  a  stack  memory  operand.  The

expression  (TWO*3)  must evaluate to an  immediate  value.  This

value  is  added  to  the contents of register SP  to  yield  the

operand's  address.  The instruction subtracts the byte  at  this

address  from  the  byte at the address specified by  the  symbol

VALUE. The result is stored at VALUE.

.pa

#4.2.6  Stack Memory Relative Operands#

Format:   <disp>(<expression>)

description:

<disp>         is  a  displacement or expression  which

must evaluate to an Immediate type  with

a  value  that  is legal for  the  32000

series microprocessor in use.

<expression>   should be an expression with SP relative

type  (see Section 2.6.8).   Parentheses

are required.

A stack memory relative operand points to a memory address  whose

double-word  value  is  added  to the displacement  to  give  the

resultant address.

Example:

SP_VAL:	.EQU	12(SP)

ADDB	R3,12(5(SP)+1)

MOVB	3(SP_VAL),R4

In  the  above  example 12(5(SP)+1) is a  stack  memory  relative

operand.   The  second assembler instruction adds register R3  to

the  specified  address.   The address is the sum of 12  and  the

double-word value at the address (5(SP+1)) ie (6(SP)). The symbol

SP_VAL  is  defined as SP relative in the  first  statement.  The

third  statement  moves a byte from the double-word at  offset  3

from SP_VAL to R4.

.pa

#4.2.7  Static Memory Operands#

Format:   <disp>(SB)

description:

<disp>         is  a  displacement or expression  which

must evaluate to an Immediate type  with

a  value  that  is legal for  the  32000

series microprocessor in use.

(SB)           specifies   the  Static  Base  register.

Parentheses are required.

A   static  memory  operand  specifies an  operand  at  a  memory

address.  The address  is  the  sum  of the displacement disp and

the contents of the Static Base register.

A  symbol may be used as a static memory operand if it has been defined

as an SB type (see Section 2.6.6).

Examples:

1.  MOVW  VALUE, 12(SB)

2.  SUBB  (TWO*3)(SB), VALUE

In example 1, 12(SB) is a static memory operand.  The instruction

moves  the  word at the address specified by the symbol VALUE  to

the word at the memory address specified by 12(SB).

In  example  2,  (TWO*3)(SB)  is a  static  memory  operand.  The

expression  (TWO*3)  must evaluate to an  immediate  value.  This

value  is  added  to the contents of register  SB  to  yield  the

operand's  address.  The  instruction subtracts the byte at  this

address  from  the byte at the address specified  by  the  symbol

VALUE. The result is stored at VALUE.

.pa

#4.2.8  Static Memory Relative Operands#

Format:   <disp>(<expression>)

description:

<disp>         is  a  displacement or expression  which

must evaluate to an Immediate type  with

a  value  that  is legal for  the  32000

series microprocessor in use.

<expression>   should  be  expression with SB  relative

type (see Section  2.6.6).   Parentheses

are required

A static memory relative operand points to a memory address whose

double-word  value  is  added  to the displacement  to  give  the

resultant address.

Example:

SB_VAL:	.EQU	12(SB)

ADDB	R3,12(5(SB)+1)

MOVB	3(SB_VAL),R4

In  the  above example 12(5(SB)+1) is a  static  memory  relative

operand.   The  second assembler instruction adds register R3  to

the  specified  address.   The address is the sum of 12  and  the

double-word value at the address (5(SB+1)) ie (6(SB)). The symbol

SB_VAL  is  defined as SB relative in the  first  statement.  The

third  statement  moves a byte from the double-word at  offset  3

from SB_VAL to R4.

.pa

#4.2.9  Program Memory Operands#

Format:   <disp>(PC)

description:

<disp>         is  a  displacement or expression  which

must evaluate to an Immediate type  with

a  value  that  is legal for  the  32000

series microprocessor in use.

(PC)           specifies   the  start  address  of  the

program  code  of  the  current  module.

Parentheses are required.

A  program  memory  operand  specifies an  operand  at  a  memory

address.   The address  is  the  sum of the displacement disp and

the start address of the program code segment of the module.

A  symbol may be used as a program memory operand if it has  been

defined as PC type (see Section 2.6.9).

Examples:

1.  MOVW  VALUE, 12(PC)

2.  SUBB  (TWO*3)(PC), VALUE

In  example  1,   12(PC)  is  a  program  memory  operand.    The

instruction moves the word at the address specified by the symbol

VALUE to the word at the memory address specified by 12(PC).

In  example  2,  (TWO*3)(PC)  is a program  memory  operand.  The

expression  (TWO*3)  must evaluate to an  immediate  value.  This

value  is  added  to the contents of register  PC  to  yield  the

operand's  address.  The  instruction subtracts the byte at  this

address  from  the byte at the address specified  by  the  symbol

VALUE. The result is stored at VALUE.

Program  memory  operands are encoded as  PC-relative  addressing

mode.   At  run time the address of a PC relative operand is  the

encoded  displacement plus the contents of the PC register.  Thus

the  Assembler  encodes  the  difference  between  the  operand's

displacement  and the value of the location counter (as a  signed

number), rather than the value of the displacement itself.

.pa

#4.2.10  Immediate Operands#

Format:   <expression>

description:

<expression>   is one of the following:

- an expression of type immediate (see Section 2.6.1)

- an expression of type long (see Section 2.6.2)

Immediate  operands are encoded into immediate  addressing  mode,

thus the operand's value is stored in the instruction stream.

The   range   of   immediate  operands  is  controlled   by   the

instruction's length specifiers as follows:

- For expressions of type immediate, the ranges are:

-128 to 255			for byte instructions.

-32768 to 65535			for word instructions.

-2147483648 to 2147483648	for double-word instructions.

- For expressions of type long, the positive ranges are:

1.28x10E-38 to 3.40x10E38	for float instructions.

2.23x10E-308 to 1.80x10E308	for long instructions.

Examples:

1.  ADDW  3, FRED

2.  SUBF  3.14159E12, F2

3.  SUBD  "MNOP", TEST

4.  ADDD  TWO*3, R4

Example  1  adds  the  word  constant 5  to  the  memory  address

specified by FRED.

Example 2 subtracts the floating point number 3.14159E12 from the

contents of register F2.

Example  3 subtracts the string constant "MNOP" from the  double-

word at address specified by TEST.

Example  4 adds the value of the expression TWO*3 to the  double-

word of register R4.

.pa

#4.2.11  Absolute Operands#

Format:  <absolute>

description:

<absolute>     is  a  constant or an  expression  which

evaluates to absolute type (see  Section

2.6.4).

An  absolute  operand specifies an operand at an absolute  memory

address.

Examples:

1.		SUBW  H'9876, @12

2.		CMPB  @ELEVEN, R6

3.  HERE:	.EQU @12

MOVW  HERE, R3

In  example  1,  @12  is an  absolute  operand.  The  instruction

subtracts the immediate operand H'9876 from the word starting  at

absolute address 12.

In example 2,  @ELEVEN is an absolute operand.  The symbol ELEVEN

must  be of immediate type.  The instruction compares the word at

the absolute address specified by ELEVEN to the low order word of

register R6.

In  example 3,  the use of @ preceding an immediate value  causes

the .EQU directive to assign HERE absolute type.  MOVW then moves

the word at the absolute  address  specified  by HERE to the low-

order word of register R3.

.pa

#4.2.12  External Operands#

Format:   <external>

description:

<external>     is an expression with external type (see

Section 2.6.5).

An  external  operand  implies both a Link  Table  entry  and  an

offset.  At run-time the contents of the Link Table entry and the

offset are added together to produce an absolute address.

Examples:

1.  ADDW  R3, EXT(4)

2.  SUBW  FIRST, EXT(FOUR) + ELEVEN

In  example 1,  EXT(4) is an external operand.   The  instruction

adds  the low order word of register R3 to the word specified  by

EXT(3).  The  external   address is the sum  of  the  double-word

contents of a Link Table entry and 0.

In  example  2,  EXT(FOUR) + ELEVEN is an external  operand.  The

instruction subtracts the word at the address specified by  FIRST

from  the word specified by EXT(FOUR) + ELEVEN.  The symbols FOUR

and ELEVEN must evaluate to immediate type.

.pa

#4.2.13  Top of Stack Operands#

Format:   TOS

description:

TOS       is the required keyword.

The  top  of stack operand specifies an operand which is  at  the

current  top  of stack,  ie the address specified by the  current

stack pointer.

TOS is a NS32000 general addressing mode.

The stack pointer is incremented by the length (in bytes) of  the

operand  after  a  read  is  performed.   The  stack  pointer  is

decremented  by  the  length (in bytes) of the operand  before  a

write  is  performed.  The stack pointer is not  modified  if  an

operand is both read and written.

Examples:

1.  ADDW  R4,TOS

2.  SUBW  TOS,TOS

Example  1  decrements the stack pointer by 2 (the  length  of  a

word)  and then adds the low-order word of register R4 to the top

of the stack.

Example  2  reads  the  word at the top  of  the  current  stack,

increments  the stack pointer by 2 (the length of a word),  reads

the  word at the new top of stack,  subtracts the first word from

the  second  word and then writes the word back over  the  second

word.

.pa

#4.2.14  Scaled-Index Byte Operands#

Format:   gen[<register>:B]

description:

gen            specifies a general operand.  It may not

be   an  immediate  operand  or  another

scaled-index operand.

[]            are the required brackets.

<register>   specifies  a  General-Purpose   register

either by using R<n>,  ie.  one of R0 to

R7,  or  by using a symbol with register

type.

:B            is the byte scaling flag.

The  resulting address of this addressing mode is the sum of  the

address  of  the general operand 'gen' and the  contents  of  the

General-Purpose register R<n> multiplied by 1.

Example:

ADDW  R4, 20(FP)[R3:B]

In  the  above  example,  20(FP)[R3:B]  is  a  scaled-index  byte

operand.  The  instruction adds the low-order word of register R4

to  the  contents of the  destination  address.  The  destination

address  is  the sum of the address specified by 20(FP)  and  the

contents of the register R3 multiplied by 1.

.pa

#4.2.15  Scaled-Index Word Operands#

Format:   gen[<register>:W]

description:

gen            specifies a general operand.  It may not

be   an  immediate  operand  or  another

scaled-index operand.

[]             are the required brackets.

<register>     specifies  a  General-Purpose   register

either by using R<n>,  ie.  one of R0 to

R7,  or  by using a symbol with register

type.

:W             is the word scaling flag.

The  resulting address of this addressing mode is the sum of  the

address  of  the general operand 'gen' and the  contents  of  the

General-Purpose register R<n> multiplied by 2.

Example:

ADDW  R4, 20(FP)[R3:W]

In  the  above  example,  20(FP)[R3:W]  is  a  scaled-index  word

operand.  The  instruction adds the low-order word of register R4

to  the  contents  of the destination  address.  The  destination

address  is  the sum of the address specified by 20(FP)  and  the

contents of the register R3 multiplied by 2.

.pa

#4.2.16  Scaled-Index Double-Word Operands#

Format:   gen[<register>:D]

description:

gen            specifies a general operand.  It may not

be   an  immediate  operand  or  another

scaled-index operand.

[]             are the required brackets.

<register>     specifies  a  General-Purpose   register

either by using R<n>,  ie.  one of R0 to

R7,  or  by using a symbol with register

type.

:D             is the double-word scaling flag.

The  resulting address of this addressing mode is the sum of  the

address  of  the general operand 'gen' and the  contents  of  the

General-Purpose register R<n> multiplied by 4.

Example:

ADDW  R4, 20(FP)[R3:D]

In the above example,  20(FP)[R3:D] is a scaled-index double word

operand.   The instruction adds the low-order word of register R4

to  the  contents of the  destination  address.  The  destination

address  is  the sum of the address specified by 20(FP)  and  the

contents of the register R3 multiplied by 4.

.pa

#4.2.17  Scaled-Index Quad-Word Operands#

Format:   gen[<register>:Q]

description:

gen            specifies a general operand.  It may not

be  an  immediate  operand  or   another

scaled-index operand.

[]             are the required brackets.

<register>     specifies   a  General-Purpose  register

either by using R<n>,  ie.  one of R0 to

R7,  or by using a symbol with  register

type.

:Q             is the quad-word scaling flag.

The  resulting address of this addressing mode is the sum of  the

address  of  the  general operand 'gen' and the contents  of  the

General-Purpose register R<n> multiplied by 8.

Example:

ADDW  R4, 20(FP)[R3:Q]

In  the above example,  20(FP)[R3:Q] is a scaled-index  quad-word

operand.   The instruction adds the low-order word of register R4

to  the  contents  of the destination  address.  The  destination

address  is  the sum of the address specified by 20(FP)  and  the

contents of the register R3 multiplied by 8.

.pa

#4.3  IMMEDIATE OPERANDS#

Immediate  operands are expressions of immediate type with values

within subranges of the full double-word range (32 bits).

#4.3.1  Quick Operands#

Format:   <quick>

description:

<quick>        is a signed constant or expression which

evaluates  to an immediate type  with  a

value in the range -8 to 7, inclusive.

A  quick  operand  is  encoded as a 4-bit  signed  integer  in  a

reserved field within the instruction itself.  Quick operands may

only be used in the quick-integer instructions. The quick operand

is  typically used to clear,  increment,  decrement or compare  a

variable.

Examples:

1.  MOVQB  -1, LAST(R4)

2.  CMPQB   2, TEST

3.  ADDQD  1, R4

Example  1  moves  the  quick operand -1 to the  operand  at  the

address specified by LAST(R4).

Example 2 compares the quick operand 2 with the operand specified

by TEST.

Example 3 adds the quick operand 1 to the double-word of register

R4.  This would effectively increment register R4.

.pa

#4.3.2  Block Length Operands#

Format:   <cons4>

description:

<const>        is  an  unsigned constant or  expression

which  evaluates  to an  immediate  type

with  a value in the range 1 to 16  (see

below).

The  block  length  operands are  4-bit  unsigned  numbers  which

specify  the  length of a block of integers.  A block may  be  no

longer than 16 bytes,  thus the allowable values that <cons4> may

take depends on the instruction's length specifier (B,  W,  or D)

as shown by the following:

Length Specifier        <cons4>

----------------        -------

B		1 to 16

W		1 to 8

D		1 to 4

The operand is multiplied by the corresponding byte length of the

length specifier and then decremented by one before being encoded

in the instruction.

The  block length operand is the final operand in  the  following

instructions:

Move Multiple      MOVMi

Compare Multiple   CMPMi

Examples:

1.  MOVMB   VAL1, R0, 4

2.  CMPMD   VAL2, R4, LENGTH/4

In example 1, the block length 4 specifies the number of bytes to

move from the address specified by VAL1 to the address  specified

by R0.

In  example 2,  the block length is the expression  LENGTH/4.  It

specifies  the  number of double-words to be  compared.  LENGTH/4

must evaluate to a positive number less than, or equal to four.

NOTE:     the  block instructions compute an address for the  two

general addressing fields,  thus the contents of R0 and

R4 are used as addresses.

.pa

#4.3.3  Bit-Field Length Operands#

Format:   <cons5>

description:

<cons5>        is  an  unsigned constant or  expression

which  evaluates  to an  immediate  type

with  a value in the range 1 to  32.  At

assembly-time,  the assembler decrements

the number by 1 before encoding it  into

the instruction.

A  bit-field  length operand is an unsigned,  5-bit number  which

specifies  the  length in a  bit-field  instruction.  The  length

operands of the 'short bit field' instructions are encoded into a

5-bit field,  and into a byte displacement in the other bit-field

instructions. The bit-field length operand is the last operand in

the following instructions:

Extract Field Short  EXTSi

Insert Field Short   INSSi

Examples:

1.  EXTSB  START, TARGET, 4, 10

2.  INSSW  BEG, END, 8, 13

In example 1,  10 is the 5-bit constant. The instruction computes

the  location of a bit field 10 bits in length and 4 bits  offset

from  the address specified by START and copies the field to  the

address specified by the symbol TARGET.

In example 2,  13 is the 5-bit constant. The instruction computes

the  destination  of  a bit field 13 bits in length  and  8  bits

offset from the address specified by END and moves the field from

the address specified by BEG to the destination.

.pa

#4.3.4  Bit-Field Offset Operands#

Format:   <cons3>

description:

<cons3>        is  an unsigned constant  or  expression

which  evaluates  to an  immediate  type

with a value in the range 0 to 7.

A  bit-field  offset  operand is a  3-bit,  unsigned  number.  It

specifies an offset which is used to calculate the location of  a

bit field. A bit-field offset operand is the third operand in the

following instructions:

Extract Field Short   EXTSi

Insert Field Short    INSSi

Examples:

1.  EXTSB  START, END, 4, 6

2.  INSSW  BEG, END, 3, 12

In example 1, 4 is a 3-bit constant. The instruction computes the

source address by adding the bit offset 4 to START and moving a 6

bit field to the destination END, the top 4 bits are zero filled.

In example 2, 3 is a 3-bit constant. The instruction computes the

target  address  by  adding  the bit  offset  3  to  the  address

specified by END, and moves a field 12 bits in length from BEG.

.pa

#4.3.5  Displacement Operands#

Format:   <disp>

description:

<disp>         is  a  displacement or expression  which

must evaluate to an Immediate type  with

a  value  that  is legal for  the  32000

series microprocessor in use.

A  displacement  operand  is the final operand in  the  following

instructions:

Return from Subroutine		RET

Return from Trap		RETT

Insert Field			EXTi

Examples:

1.  RET	4

2.  RETT	3

3.  INSW	R3, R4, 4(R3), 5

In example 1,  the displacement is 4.  It specifies the number of

bytes to remove from the stack on return from a subroutine.

In example 2,  the displacement is 3.  It specifies the number of

bytes to remove from the stack on return from a trap.

In example 3,  the displacement is 5.  It specifies the number of

bits in the bit field to be written.

Displacement  operands are encoded by the assembler in  the  same

way  as  memory displacements are.  The length of a  displacement

operand is encoded in the most significant bits of the operand in

the following way:

0XXXXXXX					1 byte

10XXXXXX XXXXXXXX				2 bytes

11XXXXXX XXXXXXXX XXXXXXXX XXXXXXXX		4 bytes

Note.  The displacement is output with the most significant bytes

first.

.pa

#4.4  PROGRAM MEMORY OPERANDS#

Format:   <label>

description:

<label>        is a label or expression which evaluates

to  a PC-relative type with a  value  in

the range -16777215 through 16777216.

A  label  operand specifies the location of  an  instruction.  At

assembly-time  the  assembler  computes the address as  a  signed

number which is the distance in bytes from the specified label to

the current instruction address.

Program  memory operands are stored in memory in the same way  as

general program memory operands (Section 4.2.9).

Branch to subroutine		BSR

Jump to subroutine		JSR

Conditional branch		B[cond]

Examples:

1.  BSR	FRED

2.  JSR	13(PC)

3.  BEQ	$+6

In example 1,  the label operand is FRED. It specifies the number

of bytes offset from the PC.

In example 2,  the displacement is 13. It specifies the number of

bytes offset from the PC.

In example 3,  the displacement is 6.  It specifies the number of

bytes offset from the beginning of the current instruction.

.pa

#4.5  REGISTER OPERANDS#

Format:   <register>

description:

<register>     is  a General-Purpose register specified

by either R<n>,  ie.  one of R0  through

R7, or a symbol with register type.

A   register   operand  specifies  one  of  the   General-Purpose

registers. The contents of the register are used as the operand.

Move				MOVi

Add				ADDi

Compare multiple		CMPMi

Register operands are equivalent to the general register operands

(see Section 4.2.1).

Examples:

1.  MOVB	R0, 12(FP)

2.  ADDW	R3, 5(SP)

3.  CMPMB	R2, FRED, LENGTH

In  example  1,  the  register operand is  R0.  The  contents  of

register R0 moved to the address computed from 12 added to the FP

register.

In  example  2,  the  register operand is  R3.  The  contents  of

register R3 is added to the word contents at the address computed

from 5 added to the current stack pointer.

In  example  3,  the  register operand is  R2.  The  contents  of

register  R2  is  compared with LENGTH bytes at  the  address  of

symbol FRED.

.pa

#4.6  REGISTER LIST (reglist) OPERAND#

Format:   [[<register>,,,]]

description:

[]             are  the  required  brackets.  A  single

level  of brackets are required even  if

no registers are specified.

<register>     specifies  a General-Purpose register or

a symbol with register type.

A   register  list  operand  may  be  used  with  the   following

instructions:

Save Registers        SAVE

Restore Registers     RESTORE

Enter New Context     ENTER

Exit Context          EXIT

Examples:

1.  FRED:	.EQU  R5

SAVE     [FRED]

2.		RESTORE  [R3]

3.		ENTER    [], 3

4.		EXIT     [R0,R1]

In example 1, the operand specifies a single register FRED, which

has  been  set to R5.  The contents of register R5 are  saved  on

stack.

In  example 2,  the operand specifies the register directly.  The

instruction removes a double-word off the stack into R3.

In example 3,  the operand specifies no registers. A displacement

of 5 is used to allocate three bytes of the stack for storage.

In example 4, the operand restores two General-Purpose registers,

R0 and R1, and then the EXIT instruction is executed.

.pa

#4.7  CONFIGURATION LIST (cfglist) OPERAND#

Format:   [[C] [,I] [,M] [,F]]

description:

[]             are the required brackets.  The brackets

must be entered even if no configuration

bit is specified.

C     specifies that a Custom Slave is installed.

I     specifies that a Interrupt Control Unit is installed.

M     specifies that a Memory Management Unit is installed.

F     specifies that a Floating Point Unit is installed.

If  one  or  more  of  these  units  are

installed, then the  user program should

set   the  corresponding  bits  in   the

NS32032  Configuration register is  set.

Otherwise,  the  bit  is  cleared.   Any

order  is  permissible as  long  as  bit

specifications  are separated by commas.

A  configuration  list  operand may be used  with  the  following

instruction:

Set Configuration Register   SETCFG

Examples:

1.  SETCFG []

2.  SETCFG [I]

3.  SETCFG [C,I,M,F]

In  example  1,  the operand specifies  no  register  bits.   The

instruction clears all bits in the Configuration register.

In example 2,  the operand specifies the I bit.   The instruction

sets the I bit and clears the F, M, and C bits.

In  example 3,  the operand specifies all bits.   The instruction

sets all bits in the Configuration register.

NOTE:     If a bit is set when the unit is not  present,  serious

hardware problems can occur.

.pa

#4.8  PROCESSOR REGISTER OPERANDS#

Format:   <proceg>

description:

<proceg>       specifies one of the following dedicated

CPU registers:

UPSR        User Processor Status Register

FP          Frame Pointer

SP          Stack Pointer

SB          Static Base

PSR         Processor Status Register

INTBASE     Interrupt Base

MOD         Module

The   function  of  the  Dedicated   CPU

Registers is described in Section 1.3.2.

The  first operand in the following instructions is  a  processor

register operand.

Load Processor Register   LPRi

Store Processor Register  SPRi

Examples:

1.  LPRD  SB, R7

2.  SPRD  INTBASE, INT_SAVE

In  example  1,  SB  is  the  Processor  register  operand.   The

instruction  copies  the  double-word in register R7  to  the  SB

register.

In  example 2,  INTBASE is the Processor register  operand.   The

instruction copies the double-word of the Interrupt Base register

to the address specified by the symbol INT_SAVE.

.pa

#4.9   MEMORY MANAGEMENT REGISTER OPERANDS#

Format:   <mmureg>

description:

<mmureg>       specifies  one of the  following  Memory

Management registers:

BPR0    Breakpoint Register 0

BPR1    Breakpoint Register 1

PF0     Program Flow 0

PF1     Program Flow 1

BCNT    Breakpoint Count

PTB0    Page Table Base 0

PTB1    Page Table Base 1

SC0     Sequential Count 0

SC1     Sequential Count 1

MSR     Memory Management Status Register

EIA     Error/Invalidate Address

Memory    Management    registers    are

described in Section 1.3.4.

The  first  operand  in the following instructions  is  a  Memory

management register.

Load Memory Management Register     LMR

Store Memory Management             SMR

Examples:

1.  LMR  EIA,	R0

2.  SMR  PF0, PFLOW_1

In example 1,  EIA is the memory management register operand. The

instruction loads the double-word in R0 into the Error/Invalidate

Address register.

In example 2,  PF0 is the memory management register operand. The

instruction  stores  the double-word contents of the PF0  at  the

address specified by the symbol PFLOW_1.

.pa

#4.10  EXTERNAL PROCEDURE OPERANDS#

Format:   <external>

description:

<external>     is  a symbol or expression with external

type and zero offset.

An  external  procedure  operand  specifies an entry in the  Link

Table.    Entries    for    external    procedure    are   called

external  procedure descriptors and consist of the 16-bit address

of  an  entry  in  the  Module Table   and   a   16-bit   offset.

During   execution   the  address of the  external  procedure  is

calculated  by  adding the offset to the address in  the   Module

Table   entry  (See  the  section  on  software  modules  in  the

Programmer's Reference Manual).

An  external  operand  is  the  only  operand  in  the  following

instructions:

Call External Procedure               CXP

Call External Procedure descriptor    CXPD

Example:

CXP   PRINTF

In the example,  PRINTF is an external operand. PRINTF references

a  Link  Table  entry address.  At assemble  time  the  Assembler

assigns a number to the PRINTF label.  At run-time the Link Table

address is extracted from the current Module Table,  the value of

the PRINTF label is multiplied by four and a double-word is  read

from  this  address.  The low-order word is the new Module  Table

address  and  the  high-order word is the  offset  to  the  start

address of the procedure being called. The offset is added to the

module's  code  address which is obtained from the  new  module's

Module Table.