.pl 62

Chapter 5

DSI-32 ASSEMBLER PSEUDO OPCODES

#5.1  INTRODUCTION#

The  DSI-32  Assembler has a variety of pseudo opcodes to  enable

the programmer to control aspects of data and code storage.

Pseudo-op			Description		 Section

---------			-----------		 -------

Symbol Creation      create a symbol, and specifies its	    5.2

type and value

Data generation	     controls the output of code and data   5.3

constants

Storage Allocation   allocates storage for run-time data    5.4

Listing Control	     controls the format of list-file	    5.5

Linkage Control	     manages the external addressing modes  5.6

Segment Control	     defines the different module segments  5.7

Module Control	     defines the module name and execution  5.8

address

Procedure Management defines the procedure structure       Ch. 6

pseudo-ops  are Assembler commands which may be embedded into the

source statements of a program. There are several types of DSI-32

assembler  pseudo-ops as can be seen from  above.  Some  generate

data,  some  generate  code  while others  only  control  listing

formats  or  how code or data is to be generated.  The  following

sections detail how the various pseudo-ops perform their tasks.

DSI-32  Assembler pseudo-ops may optionally be preceded by a  dot

(e.g. .BYTE) to conform with other 32000 assemblers.

.pa

#5.2  SYMBOL CREATION PSEUDO-OP (EQU)#

Format:   <symbol>:[: | -]  EQU  <expression>

description:

<symbol>       is  a symbol name.  The symbol name must

be followed by one colon (:), two colons

(::), or colon minus (:-).

EQU            is the pseudo-op.

<expression>   is a constant or an expression.   It may

evaluate to any type and value.  Forward

references to symbols are not permitted.

The  EQU  pseudo-op creates a symbol.  The pseudo-op  causes  the

Assembler to evaluate the expression and assign the value to  the

symbol.  The  expression  must  evaluate  to  one  of  the  types

described  in Section 2.5.   The symbol's scope may be local (:),

exported (::) or imported/exported (:-).

Once  a  symbol  has been defined,  subsequent  portions  of  the

program  may  reference that symbol directly or as a term  in  an

expression.  The  type  of the symbol restricts  the  permissible

places where it may referenced.  The first eight characters of  a

symbol's  name must be unique.  Note that in the extended version

of  the DSI-32 the first 29 characters of a symbol name  must  be

unique.  Note a  symbol's name is case dependent,  thus LABEL and

Label are different symbols.

Examples:

1  LABELA:  .EQU   12

2  LABELB::  EQU   10(FP) + LABELA

3  LABELC:  .EQU   'ABCD'

Example 1 defines the symbol LABELA. The pseudo-op assigns LABELA

the value 0000000C and the Immediate type.

Example 2 defines the global symbol LABELB. The pseudo-op assigns

LABELB the value of the expression  10(FP)+LABELA.  If LABELA has

value IM  0000000C, LABELB is assigned the value FP 00000016.

Example 3 defines the symbol LABELC. The pseudo-op assigns LABELC

the Immediate type with a value of 41424344

.pa

#5.3  DATA GENERATION PSEUDO-OPS#

The  data  generation pseudo-ops store data  in  the  instruction

stream during assembly time.

Pseudo-op     Description

---------     -----------

BYTE	 store byte-long data

SBYTE	 store signed byte-long data

WORD	 store word-long data

SWORD	 store signed word-long data

DOUBLE	 store double word-long data

SDOUBLE	 store signed double word-long data

FLOAT	 store single-precision floating point number

LONG	 store double-precision floating point number

FIELD	 store bit field

IF          conditional assembly of the following code

ENDIF       terminate appropriate IF condition

.pa

#5.3.1  BYTE pseudo-op#

Format:   [<label>] BYTE [[<repetition-factor>]] {<expression> |

<string> } ,,,

description:

<label>        an optional label.

BYTE           is the pseudo opcode.

<repetition-factor>

(optional) specifies the number of times

to  repeat the specified data byte.   It

must be an expression which evaluates to

a    non-negative    immediate     type.

<repetition  factor> must be enclosed in

square brackets "[]".

<expression>   specifies the value of the data byte. It

must evaluate to an immediate type  with

a value in the range -128 to 255.

<string>       specifies a string.  A string is entered

in  memory left to right.  A string must

fit on a single assembler line.

The  BYTE   pseudo-op generates  byte  constants.  The  Assembler

stores  the  bytes  in  the object file at  the  current  address

specified  by the Location Counter.   If  <repetition-factor>  is

greater  than one,  or more than one  expression  is   specified,

the  bytes are stored in consecutive ascending sequence  starting

at the current address.

If <string> is specified,  the ASCII contents are stored left  to

right with each character occupying one byte.

NOTE:     <expression>  may  have string constants as  terms.  In

this  case the assembler treats the ASCII value of  the

string  as a double-word integer.   The  expression  is

evaluated and the result is stored in a single byte.  A

string  used  in  a  term  may be  no  more  than  four

characters  in  length,  otherwise  an  error  will  be

reported.

.pa

Examples:

1.  0000000  PC  0C		BYTE   12

2.  0000001  PC  010101010101	BYTE   [6]  1

3.  0000007  PC  434241414243	BYTE   'CBAABC'

4.  000000D  PC  41420D	BYTE   'AB',H'0D

5.  0000010  PC  20		BYTE   '@'/2

Example 1 places 0C in a byte at address 00000000 PC.

Example  2 places 1  (repeated 6 times) in six consecutive  bytes

starting at 00000001 PC.

Example  3 places the ASCII values of 'CBAABC' in six consecutive

bytes starting at 00000007 PC.

Example 4 places the ASCII values of 'AB' starting at 0000000D PC

followed by a 0D byte.

Example  5 places the value of the expression '@'/2  at  location

00000010 PC.

.pa

#5.3.2  SBYTE pseudo-op#

Format:   [<label>] SBYTE [[<repetition-factor>]] {<expression> |

<string> } ,,,

description:

<label>        an optional label.

SBYTE          is the pseudo opcode.

<repetition-factor>

(optional) specifies the number of times

to  repeat the specified data byte.   It

must be an expression which evaluates to

a    non-negative    immediate     type.

<repetition-factor>  must be enclosed in

square brackets "[]".

<expression>   specifies the value of the data byte. It

must  evaluate to an immediate type with

a value in the range -128 to 127.

<string>       specifies a string.  A string is entered

in memory left to right.  A string  must

fit on a single assembler line.

The   SBYTE  pseudo-op  generates  signed  byte  constants.   The

Assembler  stores  the bytes in the object file  at  the  current

address  specified  by the  Location  Counter.   If  <repetition-

factor>  is greater than one,  or more than one   expression   is

specified, the bytes are stored in consecutive ascending sequence

starting at the current address.

NOTE:     <expression>  may  have string constants as  terms.  In

this  case the assembler treats the ASCII value of  the

string  as  a double-word integer.   The expression  is

evaluated and the result is stored in a single byte.  A

string  used  in  a  term may  be  no  more  than  four

characters  in  length,  otherwise  an  error  will  be

reported.

.pa

Examples:

1.  0000000  PC  FE		SBYTE   -2

2.  0000001  PC  010101010101	SBYTE   [6]  1

3.  0000007  PC  434241414243	SBYTE   'CBAABC'

4.  000000D  PC  41420D	SBYTE   'AB',H'0D

5.  0000010  PC  20		SBYTE   '@'/2

Example 1 places FE in a byte at address 00000000 PC.

Example  2 places 1  (repeated 6 times) in six consecutive  bytes

starting at 00000001 PC.

Example 3 places the ASCII values of 'CBAABC' in six  consecutive

bytes starting at 00000007 PC.

Example 4 places the ASCII values of 'AB' starting at 0000000D PC

followed by a 0D byte.

Example  5  places the value of the expression '@'/2 at  location

00000010 PC.

.pa

#5.3.3  WORD pseudo-op#

Format:   [<label>] WORD [[<repetition-factor]] {<expression> |

<string>},,,

description:

<label>        is an optional label.

WORD           is the pseudo-op name.

<repetition-factor>

(optional) specifies the number of times

to  repeat the specified data word.   It

must be an expression which evaluates to

a    non-negative    immediate     type.

<repetition-factor>  must be enclosed in

square brackets "[]".

<expression>   specifies the value of the data word. It

must evaluate to an immediate type  with

a value in the range -128 to 65535

<string>       specifies a string constant.  Unless the

string is a term in an expression it may

be no longer than two characters.

The WORD pseudo-op generates word length constants. The Assembler

stores  the  words  in the object file  at  the  current  address

specified  by  the Location Counter.   If <repetition-factor>  is

greater  than one,  or more than one  expression  is   specified,

the  words are stored in consecutive ascending sequence  starting

at the current address.  The least significant byte of a word  is

stored at first and the most significant next.

NOTE:  <expression>  may have string constants as terms.  In this

case the assembler treats the ASCII value of the string as a

double-word  integer.   The expression is evaluated and  the

result is stored in a single word.   A string used in a term

may be no more than four characters in length,  otherwise an

error will be reported.

Examples:

1.  00000000  PC  0200		WORD  8/4

2.  00000002  PC  76987698		WORD  [2] X'9876

Example  1  stores  the value of the 8/4 in a  word  starting  at

00000000 PC.

Example 2 stores the value X'9876 (repeated twice).

.pa

#5.3.4  SWORD pseudo-op#

Format:   [<label>] SWORD [[<repetition-factor]] {<expression> |

<string>},,,

description:

<label>        is an optional label.

SWORD          is the pseudo-op.

<repetition-factor>

(optional) specifies the number of times

to  repeat the specified data word.   It

must be an expression which evaluates to

a    non-negative    immediate     type.

<repetition-factor>  must be enclosed in

square brackets "[]".

<expression>   specifies the value of the data word. It

must evaluate to an immediate type  with

a value in the range -32768 to 32767.

<string>       specifies a string constant.  Unless the

string is a term in an expression it may

be no longer than two characters.

The SWORD pseudo-op generates word length signed  constants.  The

assembler  stores  the  words in the object file at  the  current

address specified by the Location Counter. If <repetition-factor>

is greater than one, or more than one  expression  is  specified,

the  words are stored in consecutive ascending sequence  starting

at the current address.  The least significant byte of a word  is

stored at first and the most significant next.

NOTE:     <expression>  may have string constants  as  terms.  In

this  case the assembler treats the ASCII value of  the

string  as  a double-word integer.   The expression  is

evaluated and the result is stored in a single word.  A

string  used  in  a  term may  be  no  more  than  four

characters  in  length,  otherwise  an  error  will  be

reported.

Examples:

1.  00000000  PC  FEFF		SWORD  -8/4

2.  00000002  PC  76987698		SWORD  [2] X'9876

Example  1  stores the value of the -8/4 in a  word  starting  at

00000000 PC.

Example 2 stores the value X'9876 (repeated twice).

.pa

#5.3.5  DOUBLE pseudo-op#

Format:   [<label>] DOUBLE [[<repetition-factor>]] {<expression> |

<string> },,,

description:

<label>        is an optional label.

DOUBLE         is the pseudo-op.

<repetition-factor>

(optional) specifies the number of times

to repeat the specified double-word.  It

must be an expression which evaluates to

a     non-negative    immediate    type.

<repetition-factor> must be enclosed  in

square brackets "[]".

<expression>   specifies the value of the  double-word.

It  must  evaluate to an immediate  type

with a value in the range -2147483648 to

2147483647.

<string>       specifies  a string  constant.  No  more

than  four  characters are allowed in  a

string constant.

The DOUBLE pseudo-op generates double-word length constants.  The

assembler  stores  the  double-words in the object  file  at  the

current   address   specified  by  the   Location   Counter.   If

<repetition-factor>  is  greater  than  one,  or  more  than  one

expression    is  specified,   the  double-words  are  stored  in

consecutive  ascending sequence starting at the current  address.

The  least significant byte of a word is stored at first and  the

most significant next.

NOTE:  <expression> may have string constants as terms.  In  this

case  the  assembler treats the ASCII value of the string as

a double-word integer.   The expression is evaluated and the

result  is stored in a word.  A string used in a term may be

no more than four characters in length,  otherwise an  error

will be reported.

Examples:

1. 00000000 PC 7654321001234567    DOUBLE  X'01234567, X'76543210

2. 00000008 PC 0505000050500000    DOUBLE  [2] 5

Example 1 stores the values X'01234567 and X'76543210 starting at

00000000 PC.

Example 2 stores the value 5 (repeated twice).

#5.3.6  SDOUBLE pseudo-op#

Format:   [<label>] SDOUBLE [[<repetition-factor>]] {<expression> |

<string> },,,

description:

<label>        is an optional label.

SDOUBLE        is the pseudo-op.

<repetition-factor>

(optional) specifies the number of times

to repeat the specified double-word.  It

must be an expression which evaluates to

a     non-negative    immediate    type.

<repetition-factor> must be enclosed  in

square brackets "[]".

<expression>   specifies  the value of the double-word.

It  must evaluate to an  immediate  type

with a value in the range -2147483648 to

2147483647.

<string>       specifies  a  string constant.  No  more

than  four characters are allowed  in  a

string constant.

The SDOUBLE pseudo-op generates double-word length constants. The

Assembler  stores  the  double-words in the object  file  at  the

current   address   specified  by  the   Location   Counter.   If

<repetition-factor>  is  greater  than  one,  or  more  than  one

expression    is  specified,   the  double-words  are  stored  in

consecutive  ascending sequence starting at the current  address.

The  least significant byte of a word is stored at first and  the

most significant next.

NOTE: <expression>  may have string constants as terms.  In  this

case the assembler treats the ASCII value of the string as a

double-word  integer.   The expression is evaluated and  the

result is stored in a word.   A string used in a term may be

no  more than four characters in length,  otherwise an error

will be reported.

Examples:

1.  00000000  PC  9CFFFFFF64000000	SDOUBLE  -100,100

2.  00000008  PC  0505000050500000	SDOUBLE  [2] 5

Example 1 stores the values -100 and 100 starting at 00000000 PC.

Example 2 stores the value 5 (repeated twice).

#5.3.7  FLOAT pseudo-op#

Format:   [<label>]  FLOAT  [[<repetition-factor>]] {<expression>},,,

description:

<label>        is an optional label.

FLOAT          is the pseudo-op.

<repetition-factor>

(optional) specifies the number of times

to  repeat the specified floating  point

number.  It  must be an expression which

evaluates  to a  non-negative  immediate

type.    <repetition-factor>   must   be

enclosed in square brackets "[]".

<expression>   specifies the value of a single floating

point   number.     Strings   are    not

permitted.  The  expression must specify

the   number  only.   No   brackets   or

operators are permissible.

The FLOAT pseudo-op generates only one single-precision floating-

point  constant per invocation.  The constants are stored at  the

current  location counter.  Symbols assigned floating point  type

are  stored internally in the assembler's symbol table as  double

precision (32 bit) numbers until they are required. Conversion to

single  precision  if necessary for a particular  instruction  is

performed automatically by the assembler.

Examples:

1  00000000 PC  DA0F4940		FLOAT  3.1415926

2  00000004 PC  00007A4600007A46	FLOAT  [2] 16000.0

Example  1   stores  the floating-point constant  3.141526  in  a

double-word starting at 00000000 PC.

Example  2  stores the floating-point constant 16000.0  (repeated

twice).

.pa

#5.3.8  LONG pseudo-op#

Format: [<label>] LONG [[<repetition-factor>]] {<expression>},,,

description:

<label>        is an optional label.

LONG           is the pseudo-op.

<repetition-factor>

(optional) specifies the number of times

to  repeat the specified floating  point

number.  It  must be an expression which

evaluates  to  a non-negative  immediate

type.    <repetition-factor>   must   be

enclosed in square brackets "[]".

<expression>   specifies the value of a single floating

point number. Strings are not permitted.

The  expression must specify the  number

only.   No  brackets  or  operators  are

permissible.

The  LONG pseudo-op generates only one double-precision floating-

point  constant per invocation.  The constants are stored at  the

current  location counter.  Symbols assigned floating point  type

are  stored internally in the Assembler's symbol table as  double

precision (32 bit) numbers until they are required.

Examples:

1.  00000000 PC  48D8124DFB210940	LONG	3.1415926

2.  00000008 PC  000000000040CF40	LONG	[2] 16000.0

000000000040CF40

Example  1 stores the floating-point constant 3.141526 in  a  two

double-words starting at 00000000 PC.

Example  2  stores the floating-point constant 16000.0  (repeated

twice).

.pa

#5.3.9  FIELD pseudo-op#

Format:   [<label>] FIELD [<subfield-size>]<subfield-value>,,,

description:

<label>        is an optional label.

FIELD          is the pseudo-op.

[<subfield-size>]

specifies  the length of the  bit  field

being   generated.   It   may   be   any

expression  which  evaluates to  a  non-

negative  immediate  type.   No  forward

referenced   symbols   are    permitted.

<subfield-size>   must  be  enclosed  in

brackets  "[]".   The subfield  size  is

required.

<subfield-value>

specifies the field value. It may be any

expression  which  evaluates to  a  non-

negative immediate type with a value  in

the  range  specified by the field  size

(e.g.,  0 to 15 for a 4-bit field,  0 to

31  for  a 5-bit  field).  The  subfield

value is required.

The FIELD pseudo-op generates one or more bit fields.  The fields

are  stored  at  the current address specified  by  the  location

counter.   Fields  are stored contiguously if there is more  than

one   <subfield-size>  <subfield-value>  pair.  Zero  filling  of

unused  bits  is performed by the assembler to the  nearest  byte

boundary.

Multiple <subfield-size> <subfield-value> pairs must be separated

by commas.

Examples:

1.  00000000 PC 0B		FIELD  [5] 11

2.  00000001 PC EB00		FIELD  [5] 11, [5] 7

Example  1 places 11 in a 5-bit field at address 00000000 PC  and

zero fills the four high orders bits.

Example  2  places  11 and 7 in two consecutive 5-bit  fields  at

address 00000001 PC and zero fills the 6 high order bits.

.pa

#5.3.10  IF pseudo-op (Enhanced assembler only)#

Format:   [<label>] IF <expression1> [<operator> <expression2>]

description:

<label>        is an optional label.

IF             is the pseudo-op.

<expression1>  the first expression which may  evaluate

to any type and a value. The value  must

not have any forward referenced symbols.

<operator>     an   operator  that   separates  the two

expressions.

<expression2>  the second expression which may evaluate

to any type and a value. The value  must

not have any forward referenced symbols.

The  IF pseudo-op enables the user to optional assemble  portions

of  the  source.  If  only <expression1>  is  supplied  then  the

assembler will evaluate the expression and if it is zero then the

all  the code until an ENDIF pseudo-op is found will be  ignored.

If  <expression1> <operator> <expression2> are all supplied  then

the  assembler will evaluate <expression1> and <expression2>  and

then  perform  the  magnitude test with the  <operator>.  If  the

result is false then the assembler will ignore all the  following

code until the corresponding ENDIF pseudo-op if found. IF - ENDIF

pairs may be nested up to 15 deep.

Note. <operator> may be one of the following: <, >, <=, >=, or =.

Examples:

1. 00000000 PC         IF   4           ;This condition true

2. 00000000 PC 1400    MOVB R0,R0

3. 00000002            IF   23+5 > 23+5 ;This is false

4.                     MOVB R0,R0       ;Don't generate code

5.                     ENDIF            ;ENDIF for second IF

6. 00000002 PC 1400    MOVB R0,R0       ;Start generating code

7. 00000004 PC         ENDIF            ;Last ENDIF for first IF

.pa

#5.3.11 ENDIF pseudo-op (Enhanced assembler only)#

Format:   [<label>] ENDIF

description:

<label>        is an optional label.

ENDIF          is the pseudo-op.

The  ENDIF  pseudo-op  terminates an IF  invocation.  Each  ENDIF

pseudo-op must be matched with one IF pseudo-op.  Thus,  they act

as  a  pair  of markers that denote the beginning and  end  of  a

portion  of  source code that is to be  conditionally  assembled.

The IF ENDIF pseudo ops ease the development of source  code

that has various different modules that are to be included in the

final product depending on various assembly time parameters.

For examples of their use see Sec 5.3.10  above.  Note  that

once  an IF is assembled that causes the code to follow it to not

be  assembled it can only be terminated by its ENDIF pair.  IF  -

ENDIF pairs may be nested up to 15 levels deep.

.pa

#5.4  STORAGE ALLOCATION DIRECTIVES#

There are five storage allocation pseudo-ops:

Pseudo-op     Description

---------     -----------

BLKB	  allocate byte storage

BLKW	  allocate word storage

BLKD	  allocate double-word storage

BLKF	  allocate double-word(s) for floating-point

storage

BLKL	  allocate quad-word(s) for long floating-point

storage

Storage allocation pseudo-ops have the following form:

[<label>] <pseudo-op> [<expression>]

<expression>  specifies  the number of elements of the  specified

length  to be allocated.   <expression> must evaluate to  a  non-

negative value of Immediate type. If symbols are used as terms in

the expression they may not be forward referenced.  No storage is

allocated  if the expression evaluates to zero.  If no expression

is specified the assembler assigns a default value of 1.

Note. The storage allocation pseudo-ops generate zero initialised

storage  in either PC relocation or SB relocation (only with  the

enhanced version of the assembler). In all other relocation modes

the space is reserved but not initialised.

The  form  of the storage allocation pseudo-ops is  described  in

subsections 5.4.1 through 5.4.5.

.pa

#5.4.1  BLKB pseudo-op#

Format:   [<label>]  BLKB  [<expression>]

description:

<label>        is an optional label.

BLKB           is the pseudo-op.

<expression>   specifies  the  number  of bytes  to  be

allocated.  It  must  be  an  expression

which  evaluates to a non-negative value

with Immediate type.

The  BLKB pseudo-op allocates <expression> bytes of  storage  for

data.  The storage is allocated from the current location counter

address.  The  storage  is  initialised to zero  if  the  current

segment is PC relocation or SB relocation (only with the enhanced

version of the assembler), otherwise the storage is allocated but

not initialised.

Examples:

1.  00000000 SB	 	BLKB   5

2.  00000005 SB	HERE:	BLKB   11

3.  00000010 SB		BLKB

4.  00000011 SB		BLKB	($-HERE)/3

In  example 1,  five bytes of storage are allocated.  The storage

starts at 00000000 SB.

In  example 2,  15 bytes of storage are  allocated.  The  storage

starts at 00000005 SB.

In  example  3,  one  byte of storage is  allocated  starting  at

address 00000010 SB.

In  example  4,  (17(SB)-5(SB))  bytes of storage  are  allocated

starting at address 00000011 SB.

.pa

#5.4.2  BLKW pseudo-op#

Format:   [<label>]  BLKW  [<expression>]

description:

<label>        is an optional label.

BLKW           is the pseudo-op.

<expression>   specifies  the  number of  words  to  be

allocated.   It  must  be an  expression

which evaluates to a non-negative  value

with Immediate type.

The  BLKW  pseudo-op allocates <expression> words of storage  for

data.  The storage is allocated from the current location counter

address.  The  storage  is  initialised to zero  if  the  current

segment is PC relocation or SB relocation (only with the enhanced

version of the assembler), otherwise the storage is allocated but

not initialised.

Examples:

1.  00000000 SB	 	BLKW   5

2.  0000000A SB	HERE:	BLKW   11

3.  00000020 SB		BLKW

4.  00000022 SB		BLKW	($-HERE)/3

In example 1,  five words of storage are allocated.  The  storage

starts at 00000000 SB.

In  example  2,  15 words of storage are allocated.  The  storage

starts at 0000000A SB.

In  example  3,  one  word of storage is  allocated  starting  at

address 00000020 SB.

In  example  4,  (22(SB)-10(SB)) words of storage  are  allocated

starting at address 00000022 SB.

.pa

#5.4.3  BLKD pseudo-op#

Format:   [<label>]  BLKD  [<expression>]

description:

<label>        is an optional label.

BLKD           is the pseudo-op.

<expression>   specifies the number of double-words  to

be allocated.   It must be an expression

which  evaluates to a non-negative value

with Immediate type.

The BLKD pseudo-op allocates <expression> double-words of storage

for  data.  The  storage is allocated from the  current  location

counter  address.  The  storage is initialised to  zeros  if  the

current  segment is PC relocation or SB relocation (only with the

enhanced  version  of the assembler),  otherwise the  storage  is

allocated but not initialised.

Examples:

1.  00000000 SB	 	BLKD   5

2.  00000014 SB	HERE:	BLKD   11

3.  00000040 SB		BLKD

4.  00000044 SB		BLKD	($-HERE)/3

In  example 1,  five double-words of storage are  allocated.  The

storage starts at 00000000 SB.

In  example  2,  15 double-words of storage  are  allocated.  The

storage starts at 00000014 SB.

In example 3, one double-word of storage is allocated starting at

address 00000040 SB.

In  example  4,   (44(SB)-14(SB))  double-words  of  storage  are

allocated starting at address 00000044 SB.

.pa

#5.4.4  BLKF pseudo-op#

Format:   [<label>]  BLKF  [<expression>]

description:

<label>        is an optional label.

BLKF           is the pseudo-op.

<expression>   specifies  the number of double-words to

be allocated.   It must be an expression

which evaluates to a non-negative  value

with Immediate type.

The BLKF pseudo-op allocates <expression> double-words of storage

for  single  precision  floating  point  data.   The  storage  is

allocated from the current location counter address.  The storage

is  initialised to zeros if the current segment is PC  relocation

or  SB  relocation  (only  with  the  enhanced  version  of   the

assembler),   otherwise   the   storage  is  allocated  but   not

initialised.

Examples:

1.  00000000 SB	 	BLKF   5

2.  00000014 SB	HERE:	BLKF   11

3.  00000040 SB		BLKF

4.  00000044 SB		BLKF	($-HERE)/3

In  example 1,  five double-words of storage are  allocated.  The

storage starts at 00000000 SB.

In  example  2,  15 double-words of storage  are  allocated.  The

storage starts at 00000014 SB.

In example 3, one double-word of storage is allocated starting at

address 00000040 SB.

In  example  4,   (44(SB)-14(SB))  double-words  of  storage  are

allocated starting at address 00000011 SB.

.pa

#5.4.5  BLKL pseudo-op#

Format:   [<label>]  BLKL  [<expression>]

description:

<label>        is an optional label.

BLKL           is the pseudo-op.

<expression>   specifies the number of quad-words to be

allocated.   It  must  be an  expression

which evaluates to a non-negative  value

with Immediate type.

The  BLKL pseudo-op allocates <expression> quad-words of  storage

for  double  precision  floating  point  data.   The  storage  is

allocated from the current location counter address.  The storage

is  initialised to zeros if the current segment is PC  relocation

or  SB  relocation  (only  with  the  enhanced  version  of   the

assembler),   otherwise   the   storage  is  allocated  but   not

initialised.

Examples:

1.  00000000 SB	 	BLKF   5

2.  00000028 SB	HERE:	BLKF   11

3.  00000080 SB		BLKF

4.  00000088 SB		BLKF	($-HERE)/3

In  example  1,  five quad-words of storage  are  allocated.  The

storage starts at 00000000 SB.

In example 2, 15 quad-words of storage are allocated. The storage

starts at 00000028 SB.

In  example 3,  one quad-word of storage is allocated starting at

address 00000080 SB.

In example 4, (88(SB)-28(SB)) quad-words of storage are allocated

starting at address 00000088 SB.

.pa

#5.5  LISTING CONTROL PSEUDO-OPS#

The  listing control pseudo-ops do not affect code generation  in

any way.   They are purely for controlling the format of the list

file produced by the DSI-32 Assembler.

Pseudo-op       Description

---------       -----------

TITLE	    defines the title to place at the top of each

listing page

SUBTITLE	    defines  the subtitle to be placed  one  line

below the title

NOLIST	    all  lines until the next LIST pseudo-op  are

not listed

LIST            restarts the listing of all lines

EJECT           force a form feed in the list file

WIDTH	    performs no function (only for compatibility)

The listing control pseudo-ops are described in subsections 5.5.1

through 5.5.6.

.pa

#5.5.1  TITLE pseudo-ops#

Format:   [<label>]  TITLE <string>

description:

<label>        is an option label.

TITLE          is the pseudo-op.

<string>       may be up to eighty characters long. The

string must be enclosed in quotes (which

are  not  printed).   The  format  of  a

string  is  the  same  as  described  in

subsection 2.4.3. The title is displayed

at the top of each listing page.

The  TITLE  pseudo-op  allows  the user  supplied  string  to  be

displayed at the top of each page of the program listing.

If  the more than one TITLE pseudo-op occurs in a program the new

string will be displayed at the top of each subsequent page.

No title is printed if no TITLE pseudo-op is specified.

Example:

TITLE  "Floating point subroutines"

The example causes the string "Floating point subroutines" to  be

displayed at the top of the current page of the program listing.

.pa

#5.5.2  SUBTITLE pseudo-op#

Format:   [<label>]  SUBTITLE <string>

description:

<label>        is an optional label.

SUBTITLE       is the pseudo-op.

<string>       may   be  up  to  eighty  characters  in

length.  The string must be enclosed  in

quotes  (which  are not  printed).   The

format  of  the string is  the  same  as

described   in  subsection  2.4.3.   The

subtitle is displayed at the top of each

page after the title (if there is one).

The SUBTITLE pseudo-op enables the user to specify a string to be

displayed  at the top of each page of the  program  listing.  The

subtitle will be displayed below the title, if specified.

The  SUBTITLE pseudo-op will cause the string to be displayed  at

the  top of the current page if it is the first statement on  the

line, otherwise only subsequent page are affected.

If  more  than  one SUBTITLE pseudo-op occurs in a  program  each

occurrence will affect only the subsequent pages.

No subtitle is displayed if no SUBTITLE pseudo-op is specified.

Example:

SUBTITLE  "Subroutine divide"

The  example causes the string "Subroutine divide" to be  printed

at  the top of the current page if it is the first  statement  on

the page, otherwise it is printed at the top of the next page.

.pa

#5.5.3  NOLIST pseudo-op#

Format:   [<label>]  NOLIST

description:

<label>        is an optional label.

NOLIST         is the pseudo-op.

The  NOLIST  pseudo-op causes the assembler to stop  listing  all

subsequent  lines.  The lines are assembled,  but not directed to

the listing file.  Error messages will still be displayed.

Example:

NOLIST

ADDB	0(SB),R5

MULW	R3,MATHS

JSR	SPECIAL

LIST

In  the  example the NOLIST pseudo-op suppresses the printing  of

three  lines of text.  Listing of text is restarted by  the  LIST

pseudo-op.

.pa

#5.5.4  LIST pseudo-op#

Format:   [<label>]  LIST

description:

<label>        is an optional label.

LIST           is the pseudo-op.

The  LIST  pseudo-op  restarts the listing  of  assembler  source

lines.   The listings of source lines is suppressed by the NOLIST

pseudo-op.

Example:

NOLIST

ADDB	0(SB),R5

MULW	R3,MATHS

JSR	SPECIAL

LIST

NXT:	BEQ	DONE

In the example the LIST pseudo-op restarts the listing after  the

which was stopped by the NOLIST pseudo-op.

.pa

#5.5.5  EJECT pseudo-op#

Format:   [<label>]  EJECT

description:

<label>        is an optional label.

EJECT          is the pseudo-op.

The  EJECT pseudo-op causes the assembler to force a form feed to

a  new page.   The statement containing the EJECT pseudo-op  does

not appear on the listing.

.pa

#5.5.6  WIDTH pseudo-op#

Format:   [<label>]  WIDTH  <expression>

description:

<label>        is an optional label.

WIDTH          is the pseudo-op.

<expression>   must evaluate to an immediate type.

Note.  This  pseudo-op performs no function and is  included  for

compatibility with other 32000 assemblers.

.pa

#5.6  LINKAGE CONTROL PSEUDO-OPS#

The  linkage control pseudo-ops enable the programmer to  specify

which  symbols  are global and which symbols are external to  the

module being assembled.

Pseudo-op	Description

---------	-----------

IMPORT	specifies external symbols

IMPORTP	specifies external procedure symbols

EXPORT	specifies public symbols

EXPORTP	specifies public procedure symbols

COMM         specifies public symbol of given length

A  public symbol is a symbol that has been defined by  an  EXPORT

pseudo-op,  or an EXPORTP pseudo-op or double colon (::). It must

have PC, ABS or SB relocation type.

An  external  symbol  is a symbol that has been defined  with  an

IMPORT pseudo-op,  IMPORTP pseudo-op or colon minus (:-).  It  is

assigned  EXT relocation type,  and refers to a symbol in another

module.  Every  external  symbol must correspond to  exactly  one

public symbol.

A public symbol with a specific name may be exported by only  one

module.   The  same symbol may be imported by many  modules.  The

linker  satisfies  the public and external symbol  references  at

link time.  An error occurs if an imported symbol has no matching

exported symbol.

A symbol may be both exported and imported by the same module. To

all  other  modules the symbol behaves as though it was  exported

only.  To the module in which it was exported, though, the linker

builds  a link table entry for the symbol in the  current  module

table.  This  allows  the  symbol to be recursively  called  from

the module in which it is defined.

Subsections  5.6.1  through 5.6.4 describe  the  linkage  control

pseudo-ops.

.pa

#5.6.1  IMPORT pseudo-op#

Format:   IMPORT <symbol>,,,

description:

IMPORT         is the pseudo-op.

<symbol>       is  the  name of a  public  symbol.  The

symbol must be defined in another module

and  it  must  have  PC,   SB,   or  ABS

relocation  type.  More than one  symbol

may be specified per line,  each  symbol

must be separated by a comma.

The  IMPORT  pseudo-op defines external symbols  which  reference

data in some other module.  The IMPORT pseudo-op must not be used

to  import  external  procedures,   the  IMPORTP  pseudo-op  (see

subsection 5.6.2) should be used.

The  symbol  is  entered  into the assembler's  symbol  table  at

assembly time.   It is assigned EXT type, an offset of zero and a

Link  Table number which starts at zero and increments  for  each

imported  symbol whether it is imported by the IMPORT or  IMPORTP

pseudo-ops.

NOTE:     The  line on which the symbol is imported  must  appear

before  any  other statement which references the  same

symbol.  Otherwise,  an  error will be reported by  the

assembler.

Since  the external symbol references are only resolved

at link time,  only then will an error be reported  for

mismatched symbols.

Example:

IMPORT	ONE, TWO, THREE

In  the example if the symbols are the first imported symbols  in

the program then,  ONE is assigned link table entry zero,  TWO is

assigned one and THREE is assigned two.

.pa

#5.6.2  IMPORTP pseudo-op#

Format:   IMPORTP <symbol>,,,

description:

IMPORTP        is the pseudo-op.

<symbol>       is  the  name of a  public  symbol.  The

symbol must be defined in another module

and  it  must  have  PC,   SB,   or  ABS

relocation  type.  More than one  symbol

may be specified per line,  each  symbol

must be separated by a comma.

The  IMPORTP  pseudo-op defines external symbols which  reference

external procedures in some other module.  The IMPORTP  pseudo-op

must  not  be  used to import data,  the  IMPORT  pseudo-op  (see

subsection 5.6.1) should be used.

The  symbol  is  entered into the  assembler's  symbol  table  at

assembly time.   It is assigned EXT type, an offset of zero and a

Link  Table  number which starts at zero and increments for  each

imported  symbol whether it is imported by the IMPORT or  IMPORTP

pseudo-ops.

NOTE:     The  line  on which the symbol is imported must  appear

before  any other statement which references  the  same

symbol.   Otherwise  an  error will be reported by  the

assembler.

Since the external symbol references are only  resolved

at  link  time,only then will an error be reported  for

mismatched symbols.

Example:

IMPORT	ONE, TWO, THREE

In  the example if the symbols are the first imported symbols  in

the program then,  ONE is assigned link table entry zero,  TWO is

assigned one and THREE is assigned two.

.pa

#5.6.3  EXPORT pseudo-op#

Format:   EXPORT <symbol>,,,

description:

EXPORT         is the pseudo-op.

<symbol>       is the name of a symbol which is defined

in the current module.  It must have one

of the following relocation  types:  PC,

ABS,  or SB. More than one symbol may be

exported  per line,  each symbol must be

separated by a comma.

The  EXPORT pseudo-op enables a symbol to be given public  scope.

The symbol may then be imported by an IMPORT pseudo-op in another

module.  A  symbol which has been exported from a module  may  be

used   in  that  module  as  an  instruction  operand  or  in  an

expression.  The  EXPORT  pseudo-op must not be  used  to  export

procedures.  The  EXPORTP pseudo-op (see subsection 5.6.4) should

be used.

A  symbol  may  be exported by following its  definition  with  a

double-colon (::).   If the double colon (::) is used without the

PROC  pseudo-op (Section 6.2),  then the symbol is assumed to  be

referencing data.

NOTE:     A symbol which appears after the EXPORT statement  must

also  be defined in some statement following the EXPORT

statement.  Otherwise an error will be reported by  the

assembler.

Example:

EXPORT	ONE, TWO, THREE

In the example three symbols are assigned public scope:  ONE, TWO

and THREE.

.pa

#5.6.4  EXPORTP pseudo-op#

Format:   EXPORTP <symbol>,,,

description:

EXPORT         is the pseudo-op.

<symbol>       is the name of a symbol which is defined

in the current module.  It must have one

of  the following relocation types:  PC,

ABS,  or SB. More than one symbol may be

exported per line,  each symbol must  be

separated by a comma.

The EXPORTP pseudo-op enables a symbol to be given public  scope.

The  symbol  may  then  be imported by an  IMPORTP  pseudo-op  in

another  module.  A symbol which has been exported from a  module

may  be  used in that module as an instruction operand or  in  an

expression.  The  EXPORTP  pseudo-op must not be used  to  export

data. The EXPORT pseudo-op (see subsection 5.6.3) should be used.

A  symbol  may  be exported by following its  definition  with  a

double-colon  (::).   If the double colon (::) is used  with  the

PROC  pseudo-op (Section 6.2),  then the symbol is assumed to  be

referencing an external procedure.

NOTE:     A symbol which appears after the EXPORTP statement must

also be defined in some statement following the EXPORTP

statement.  Otherwise  an error will be reported by the

assembler.

Example:

EXPORTP	ONE, TWO, THREE

In the example three symbols are assigned public scope:  ONE, TWO

and THREE.

.pa

#5.6.5 COMM pseudo-op (Enhanced assembler only)#

Format:   COMM   <symbol>,length

description:

COMM           is the pseudo-op.

<symbol>       is the  name of a symbol which is to  be

imported  from  some other  module.  The

symbol refers to a data item.

length         is the length of the data item.  If  the

symbol  is  not  defined  in  any  other

module  via  an export then  the  linker

will  assign  the variable and  allocate

enough  static base data to satisfy  the

length specifier.  The data item is also

zeroed by the linker.

The  COMM  pseudo-op  enables a data item  to  be  accessed  from

another  module or to be created by the linker at link time.  The

COMM  pseudo op enables the assembler to support the  concept  of

named common such as required by languages such as Fortran.

Example:

COMM   FRED,10

In  the  example the data item FRED is imported from some  module

and  its length is 10 bytes.  If FRED is satisfied by some  other

module  at  link time then the length specifier will  be  ignored

else  the linker will create a dummy module and allocate 10 bytes

for  the data item.  The linker will also create an entry in  the

appropriate Link Table.

.pa

#5.7  SEGMENT CONTROL PSEUDO-OPS#

The  segment control pseudo-ops enable the programmer to  control

the  Assembler's  current relocation mode and the  value  of  the

location counter. The segment control pseudo-ops are described in

detail below:

pseudo-op	   Description

---------	   -----------

DSECT	   location counter set to IM relocation mode

PROGRAM	   location counter set to PC relocation mode

STATIC	   location counter set to SB relocation mode

LOC	   location counter set to the specified value

ENDSEG	   location counter and relocation mode set to

the values prior to the current segment

ALIGN	   location counter set to the specified offset

The  segment  control pseudo-ops enable the programmer to  define

various  program  segments.  Each segment contains data  or  code

relative to the segment base.  The various data or code  segments

are assigned contiguous addresses by the assembler, the FP and SP

segments  are  the only exception to this rule.  Since  they  are

actually  relative to the stack they only exist for the  duration

of a procedure invocation etc.

The  form  and description of the segment control  pseudo-ops  is

described in subsections 5.7.1 through 5.7.6.

.pa

#5.7.1  DSECT pseudo-op#

Format:   DSECT

description:

DSECT          is the pseudo-op.

The DSECT pseudo-op defines a dummy segment. This enables symbols

to  be  assigned  immediate type and a  particular  value.  These

symbols  may  then be used within expressions or as  operands  in

instructions.  These  symbols may only be referenced  after  they

have been defined in a segment.

The  location  counter  is  set  to an offset of  0  and  the  IM

relocation  mode.   No object code is produced for  this  segment

type.

Example:

DSECT

ONE:	BLKB	1

TWO:	BLKB	2

THREE:	BLKB	3

ENDSEG

In  the  example three symbols are assigned Immediate  type.  The

symbols  ONE,  TWO  and  THREE are assigned values  0,  1  and  3

respectively.

The symbols may then be used to reference data.  Since their type

is  Immediate they may be used as displacements in  frame  memory

operands. The following example shows one use of these symbols:

ADDB	1,ONE(FP)

SUBB	2,TWO(FP)

MULD	3,THREE(FP)

.pa

#5.7.2  PROGRAM pseudo-ops#

Format:	PROGRAM

description:

PROGRAM		is the pseudo-op.

The  PROGRAM  pseudo-op  defines a  program  code  segment.   The

instructions and constant data is stored in this segment.

The  location counter is set to PC relocation mode the offset  to

the  next  available PC address (initially  zero).  The  location

counter is incremented after every instruction.

The program code segment is the only segment in which object code

is generated. All other segments only reserve storage relative to

one of the segment bases.

Example:

PROGRAM

In the example the Location Counter is set to PC relocation mode.

The offset is set to the next PC address.

.pa

#5.7.3  STATIC pseudo-op#

Format:   STATIC

description:

STATIC         is the pseudo-op.

The  STATIC pseudo-op defines a static base segment.  The  static

base segment is relative to the SB register,  which is automatic-

ally loaded when a module is first entered. Forward references to

symbols defined in static base segments are allowed.

The  symbols in a static base segment are assigned  monotonically

increasing addresses which start at 0 for the first symbol in the

first static base segment.   The location counter for the  static

base  relocation mode may not be decremented thus it is saved  on

entering a different relocation mode.

The  static  base segment contains storage allocation  pseudo-ops

only.   No  instructions or pseudo-ops which generate code  (e.g.

DOUBLE, FIELD, etc) may be used.

Example:

STATIC

ONE:	BLKB	1

TWO:	BLKB	2

THREE:	BLKB	3

ENDSEG

In  the  example  three  symbols are defined  in  a  static  base

segment.  The  STATIC  pseudo-op sets the location counter to  SB

relocation mode and to the next available SB address.

.pa

#5.7.4  LOC pseudo-op#

Format:   LOC [<expression>]

description:

LOC		is the pseudo-op.

<expression>   defines   a  new  relocation  mode   and

offset.  The expression must evaluate to

a  value with IM,  PC,  SB,  FP,  or  SP

relocation mode.

The  LOC  pseudo-op  causes a new  segment  to  be  entered.  The

location   counter   is  set  to  the  value  specified  by   the

expression.

If  the  expression evaluates to SP or FP  relocation  mode,  any

value is allowed.  If the relocation mode is IM,  PC,  or SB, the

expression's value must evaluate to a value greater than or equal

to  the  last value for the particular relocation  mode.  If  the

Relocation  Mode  is PC,  the  assembler will output  zero  value

bytes  till the location counter reaches the address specified by

<expression>.

The location counter is incremented after each instruction in IM,

PC and SB relocation modes.  The location counter is  decremented

in the FP and SP relocation modes.

Examples:

1.	LOC  12

2.	LOC  12(FP)

3.	LOC  12(PC)

Example  1 assigns IM relocation mode and an offset of 12 to  the

location counter.

Example  2 assigns FP relocation mode and an offset of 12 to  the

location counter.

Example  3 assigns PC relocation mode and an offset of 12 to  the

location counter.

.pa

#5.7.5  ENDSEG pseudo-op#

Format:   ENDSEG

description:

ENDSEG         is the pseudo-op.

The  ENDSEG  pseudo-op  cause  the current  segment  to  end  and

restores  the  location counter relocation mode and value it  had

before  the  current segment was entered by one  of  the  segment

pseudo-ops (see subsections 5.7.1 through 5.7.4).

Each  DSECT,  PROGRAM,  STATIC  and  LOC pseudo-op  must  have  a

matching ENDSEG pseudo-op.

NOTE:     The  assembler  assigns  a default value  and  type  of

00000000  PC to the location counter at the start of an

assembly  thus  a  program	may  have  no  opening

pseudo-op and no ENDSEG and still be valid.

Example:

00000000 PC			     1  	PROGRAM

00000000 PC CEE0D6C0000000C00000     2  	MULB  TEST1,TEST2

00000000 SB			     3  	STATIC

00000000 SB			     4  TEST1:	BLKB	4

00000004 SB 			     5  	ENDSEG

0000000B PC 0C00		     6  TEST2:	WORD	12

0000000D PC CEFCD6007E		     7  	DIVB  TEST1,TEST2

00000012 PC 			     8  	ENDSEG

00000000 FP 			     9  	LOC	0(FP)

FFFFFFF6 FP			    10  TEST3:	BLKB	10

FFFFFFF6 FP 			    11  	ENDSEG

In  the  example  the  ENDSEG  pseudo-op  terminates  the   three

segments: PC, SB and the FP segments.

.pa

#5.7.6  ALIGN pseudo-op#

Format:   ALIGN <expression1> [,<expression2>]

description:

ALIGN          is the pseudo-op.

<expression1>	 defines  the  base  of  a  new  location

counter   value.   The  expression  must

evaluate  to  a  non-negative  value  of

Immediate  type.   No forward references

to symbols are permitted.

<expression2>	 optional  expression which  defines  the

offset   of  the  new  location  counter

value. The expression must evaluate to a

non-negative value of Immediate type and

must  be  less  than  <expression1>   in

value. If <expression2> is not specified

it takes on a default value of zero (0).

No  forward  references to  symbols  are

permitted.

The  ALIGN  pseudo-op  enables the location counter value  to  be

changed while still maintaining the current relocation mode.  The

new  value  is the sum of a multiple of  <expression1>,  and  the

<expression2> and is always equal to or greater than the  current

location counter.  The value satisfies the following definition:

new value MODULUS  <expression1> = <expression2>

If  only <expression1> is specified,  the new value is a multiple

of <expression1> only. The new value is the multiple of the basis

that  is  greater than or equal to the current location  counter.

For  example,  if  <expression1> is 8 and  the  current  location

counter is 25, then the new value is 32 (i.e. 4*8).

If  both <expression1> and <expression2> are specified,  the  new

value is the sum of the multiple of the basis and the offset. For

example,  if  <expression1>  is 8,  <expression2> is 5,  and  the

current  location counter is 25,  then the new value is 29  (i.e.

3*8+5).

NOTE:     The ALIGN pseudo-op may only be used when assembly mode

is  standard.   If the ALIGN pseudo-op is used in a  PC

relocation  segment,  the Assembler will zero-fill  all

bytes  between  the  current  PC address  and  the  new

address     calculated    from    <expression1>     and

<expression2>.

.pa

Example:

00000000 SB			   1		STATIC

00000000 SB                        2  TEST1:	BLKW	1

00000002 SB			   3		ALIGN   5

00000005 SB			   4  TEST2:	BLKW

00000007 SB			   5		ALIGN   12, 3

0000000F SB			   6  TEST3:	BLKW

00000011 SB			   7		ENDSEG

In  the  example two ALIGN pseudo-ops  appear.  In  line  3,  the

pseudo-op sets the location counter to a multiple of 5. Since the

location counter is 00000002  SB,  the new value will be 00000005

SB i.e., 1*5. In line 5, the pseudo-ops sets the location counter

to a multiple of 12 plus 5. Since the current location counter is

7, the the new value will be 15 i.e., 1*12+3.

.pa

#5.8  MODULE CONTROL PSEUDO-OPS#

There   are  two  module  control  pseudo-ops  which  enable  the

programmer  to  specify  the module name  and  the  module  start

address:

pseudo-op		Description

---------		-----------

MODULE		defines the module's name

START		defines the module's execution address

Subsections  5.8.1 and 5.8.2 describe the module control  pseudo-

ops.

#5.8.1  MODULE pseudo-op#

Format:   MODULE <symbol>

description:

MODULE         is the pseudo-op.

<symbol>       defines the name of the current module.

The MODULE pseudo-op defines the name of the current module. Only

one  MODULE pseudo-op is allowed per assembler source  file.  The

pseudo-op may appear anywhere in the file. The name of the source

file  is  used  as  the module name if  no  MODULE  pseudo-op  is

specified.

Example:

MODULE	MSUBRT

In the example the symbol MSUBRT is assigned as the module name.

.pa

#5.8.2  START pseudo-op#

Format:   START [<expression>]

description:

START          is the pseudo-op.

<expression>   specifies the module's execution address.

The  START pseudo-op specifies where execution is to  start.  The

expression must evaluate to a value with PC relocation type.   If

a  START pseudo-op is not specified,  the assembler defaults  the

start address to 00000000 PC.

The START pseudo-op assigns the module as the MAIN module.