.pl 66

DEFINICON SYSTEMS, INC.

DSI-32 COPROCESSOR BOARD

& MS/PC DOS SYSTEM SOFTWARE

C RUNTIME LIBRARY

and

COMPILER REFERENCE MANUAL

part# DOC20060-1.1

.pa

SECTION 1

Getting Started with C

1.1 System Requirements

To  operate the Definicon Systems' C compiler,  your system  must

satisfy the following requirements:

o   A DSI-32000 coprocessor card,  with a minimum of 256K  bytes

of memory.

o   An IBM compatible clone with a minimum of 96K bytes of  free

memory  exclusive  of  operating system  requirements  (256K

minimum of memory recommended).

o   Enough  disk space to contain the compiler,  the program  to

compile  and the output file (can be run  from  floppy,  but

hard disk highly recommended).

1.2 Runtime Requirements

To execute the compiled,  assembled and linked programs you  will

need the following computer resources.

o   A DSI-32000 coprocessor card,  with a minimum of 256K  bytes

of memory.

o   An IBM compatible clone  with a minimum of 96K bytes of free

memory exclusive of operating system requirements.

Note   that   both  the  system  requirements  and  the   runtime

requirements are for an absolutely minimum system.  A DSI-32 card

with only 256K bytes of memory will severely restrict the size of

functions  that  can be compiled.  1M byte of memory will  enable

very large functions to be compiled.  Also due to the size of the

C  compiler (around 200k bytes of executable code),  a hard  disk

drive,  or  at  the very least two floppy disk  drives,  will  be

particularly useful in retaining your sanity.

1.3 C Components

The  Definicon  Systems' C compiler package consists  of  several

components, some executable.  The executable components are the C

compiler,  the  assembler  and  the linker.  The  C  system  also

includes  six  special files that you can include in a C  program

with  the  C '#include' directive.  These  special  files  enable

programs  to be developed in a more portable fashion.   Table 1.1

details the various components that are a part of the C system.

.pa

Table 1.1 C System components

-----------------------------------------------------------------

|  Component  |  Disk File  |           Description             |

-----------------------------------------------------------------

|  compiler      CC.E32        the compiler module              |

|                CC.CMD        the CP/M command parser          |

|                CC.EXE        the MS/DOS command parser        |

|                LOAD.CMD      the CP/M program loader          |

|                LOAD.EXE      the MS/DOS program loader        |

|                                                               |

|  assembler     AS.E32 &      the enhanced compiler compatible |

|                AS.CMD/AS.EXE assembler                        |

|                AS32032.OPC   the opcode file for the assembler|

|                                                               |

|  linker        LN.E32 &      the enhanced compiler compatible |

|                LN.CMD/LN.EXE linker                           |

|                                                               |

|  #include      STDIO.H       various system dependent equates |

|  files         PORTAB.H      various portable definitions     |

|                ERRNO.H       definitions of various errors    |

|                MTH.H         definitions of various numbers   |

|                SETJMP.H      non-local program jump routine   |

|                CTYPE.H       various ASCII routines           |

|                                                               |

|  libraries     CRTLIB.O32    standard C functions (see runtime|

|                              section)                         |

|                CLIB.O32      low level C routines             |

|                MATH.O32      C math functions                 |

|                                                               |

|  sample        HELLO.C       simple program                   |

|  programs      README.DOC    updated notes on C system and    |

|                              documentation                    |

-----------------------------------------------------------------

.pa

1.4 Minimum Working Disk

To  compile  a program,  a diskette with the  following  programs

forms a minimum configuration:

o   CC.E32                 the compiler

o   CC.EXE or CC.CMD       the command parser for MS/DOS or C/PM

o   LOAD.EXE or LOAD.CMD   the DSI-32 program loader

o   32IO.E32               the DSI-32 communications module

o   any required .H files

To  assemble  and  link  the  program  ready  for  execution  the

following programs form a minimum configuration:

o   AS.E32                 the assembler

o   AS.CMD or AS.EXE       the command parser for the assembler

o   AS32032.OPC            the opcode file for the assembler

o   LN.E32                 the linker

o   LN.CMD or LN.EXE       the command parser for the linker

o   CRTLIB.O32             C runtime library

o   CLIB.O32               low level C functions

o   MATH.O32               C math functions

o   LOAD.EXE or LOAD.CMD   the DSI-32 program loader

o   32IO.E32               the DSI-32 communications module

o   LNC.D32                default file that controls the linker

and   specifies  the   libraries   to

search.

.pa

1.5 Compiling the demonstration program

The  following  section describes how to  compile,  assemble  and

link  the  demonstration  program  supplied  on  the  C  Language

diskette.  The instructions assume that you are familiar with the

operation of your operating system whether it is CP/M or MS/DOS.

Prior to following these instructions be sure to make a backup of

your  master  C diskette(s) and store the original(s) in  a  safe

place.

1)   Create  a C working disk,  and an  assembler/linker  working

disk with the files as shown in Section 1.4.  If your system

has  a  hard disk then just move all the files  from  the  C

system diskette(s) to an area on the hard disk.

With  your operating system disk in drive A and your C  work

disk  in  drive  B  which contains a  copy  of  the  HELLO.C

program,  you  are  ready  to compile and test  your  sample

program.  To look at the program you can type the program as

follows:

B>TYPE HELLO.C

main()

{

printf("Welcome to the world of 32 bit computers");

}

2)  Now we are ready to compile the program.

B>CC HELLO

Note that it is not necessary to enter the .C extension when

invoking the compiler, the command parser performs that task

automatically. When the compiler has loaded and successfully

started  compiling your program it will display  its  signon

message.  At  the  end of the compilation the compiler  will

display any errors that have occurred and then return you to

the  operating  system prompt.  If no errors  have  occurred

then you are ready to assemble and link the program.

3)  Now we are ready to assemble the program.

B>AS HELLO

Note that as for the compiler,  the assembler  automatically

searches  for  the .A32 extension.  Once the  assembler  has

successfully loaded,  it will display its signon message. As

each assembly pass progresses, the pass will be displayed on

the  console.  Any errors that are detected are displayed on

the console as they occur.

.pa

4)  Now we are ready to link the program.

B>LN FILE=HELLO

The  linker  will link the program with the C libraries  and

produce a file that is executable on the DSI-32  board.  The

executable file will have a .E32 extension.

5)  Now we can load and run the program.

B>LOAD HELLO

Welcome to the world of 32 bit computers

B>

Note  that this example concentrated on linking a  single  C

program,  but  in  the  real world a typical C  program  may

consist of several separately compiled modules.  Below is an

example  of  how to link several modules to  form  a  single

executable  module.  In addition we will assume that we have

our own library, mylib.o32, that we want to search.

A>LN FILE=file1,file2,file3,file4 LIB=mylib

The linker will first read all the symbol information  about

the  files into its tables and then after satisfying as many

of  the symbols as possible it will then search any  of  the

user   specified  libraries  to  resolve  any  import/export

references that have been left unresolved.

Note  that to build a user library you must have  access  to

the Definicon librarian, LIB.

.pa

SECTION 2

Operating the C compiler

2.1 General Overview

The  Definicon  Systems' C compiler is a single  pass  optimizing

compiler.  It  performs automatic register optimization and  will

put  variables and pointers into registers depending  on  various

parameters  such  as the lifetime of a variable and its  intended

use (e.g.  if the address of a variable is taken at some stage in

a  module,  then  the compiler will not place the variable  in  a

register).  The C compiler supports all the standard features  of

Berkley 4.2 Unix portable C,  including bit fields and enumerated

data types.

- The sizes of various C variables are listed below:

unsigned char            8 bits

char            8 bits

unsigned short int      16 bits

short int      16 bits

unsigned int            32 bits

int            32 bits

unsigned long           32 bits

long           32 bits

pointer        32 bits

float          32 bits

double         64 bits

- Program  size and data size are limited only by  the  available

memory;  small model, medium model, large model, etc. limitations

of  8088/8086/80286  IBM-PC clone systems do not exist  with  the

National Semiconductor's 32000 series of 32 bit microprocessors.

2.2 Compiler Command Line

The  command line invokes the C compiler,  specifies  the  source

file  to  compile  and  passes any special  instructions  to  the

compiler.  The maximum compiler command line is 128 characters in

length.  The form of the command line is shown below:

B>CC    option_switches    source_filename

Note that the .C filetype extension does not have to be  supplied

since the parser will automatically append the correct extension.

If  an  extension is supplied in the command line then that  file

will  be  compiled regardless of the .C convention.  Also  if  no

extension is supplied on the command line and a file that has  no

extension  but  has  the  right name  exists,  then  it  will  be

compiled.

.pa

2.3 Stopping the Compiler

The  C  compiler  may be stopped at any stage of  compilation  by

entering a <CTRL>C at the console (hold the CTRL key down and hit

the C key). The compiler will immediately terminate and exit back

to the operating system.  Note that all programs that run on  the

DSI-32  card  can be aborted in this way,  as well as  assembler,

linker and user programs.  Upon termination,  all files that  are

open  are automatically closed.  Note also that any output  files

that  were open will be closed in whatever state they were in  at

the time <CTRL>C was entered. The output files will not be erased

but they will probably contain incomplete data.

2.4 Error Messages

All  error  messages are displayed during the  compilation.   The

line number in which the error was detected and the type of error

are displayed on the console.

2.5 Compiler Command Line Options

Command  line option switches enable the user to control  certain

functions of the compiler.  An option switch always starts with a

minus  sign  and  follows with  certain  alphanumeric  sequences.

Spaces  must  not be left between the any of the characters  that

make up an option switch but at least one space must separate  an

option  switch from any other parameter.  Table 2.1  details  the

available options and their function.

Command line options are entered as follows:

B>CC -O1 -O2 -DDEBUG=OFF PROGRAM

#Note#

There is no case sensitivity for the command line  options.

The command line parser CC.CMD or CC.EXE converts symbol names to

lowercase,  this  means that the above command line will  produce

the  directive  to the compiler "#define debug =  off",  NOT  the

desired directive "#define DEBUG = OFF".

.pa

-----------------------------------------------------------------

|    Option    |              Description                       |

-----------------------------------------------------------------

|    -c            causes comments to be output with the pre-   |

|                  processor output                             |

|                                                               |

|    -dname        define "name" to the preprocessor with the   |

|                  value 1.  Equivalent to "#define name 1"     |

|                                                               |

|    -dname=string define "name" to the preprocessor with the   |

|                  value "string".  Equivalent to entering      |

|                  "#define name string" at the top of the file |

|                                                               |

|    -e            do not compile the program, just place the   |

|                  output from the preprocessor onto the console|

|                                                               |

|    -istring      included filenames that do not start with "/"|

|                  are searched for in the directory "string"   |

|                                                               |

|    -o1           attempt to optimize the program to be as fast|

|                  possible.  The Definicon System's C compiler |

|                  generally performs many optimizations that   |

|                  are normally options on other compilers, so  |

|                  it may be counter productive to use this     |

|                  optimization switch as the code produced is  |

|                  around 10% larger.  Experimenting with this  |

|                  option will allow you to determine when it   |

|                  will prove to be useful                      |

|                                                               |

|    -o2           allow the optimizer to assume that memory    |

|                  locations do not change except by explicit   |

|                  stores.  That is, the optimizer is guaranteed|

|                  that no memory locations are I/O device      |

|                  registers that can change asynchronously     |

|                                                               |

|    -uname        undefine the predefined preprocessor symbol  |

|                  "name".  Equivalent to "#undef name".        |

|                                                               |

|    -x55          make bit fields of int, short and char be    |

|                  signed.  The default is unsigned.            |

|                                                               |

|    -x56          allocate each enum type as the smallest size |

|                  which allows representation of all listed    |

|                  values (i.e. from the list "char", "short",  |

|                  "int", "unsigned char", "unsigned short" or  |

|                  "unsigned"). The default is to allocate "int"|

-----------------------------------------------------------------

.pa

Note.  For  information  on the C compiler and runtime  libraries

refer to the Definicon C reference manual.

nted in STDIO.H, therefore it must not

be  declared in the user source.

#Declarations#

int       num;       -- can be any number.

int       aret;      -- may be of any type.

#Syntax#

aret  =   abs(num);

.pa

##atoi,_atof,_atol##

The   atoi,   atof  and  atol  functions  take  an  ASCII  string

representing  a  number and convert the string into  an  integer,

double floating point or long integer number, respectively.

#Declarations#

char      *string;    -- pointer to a null terminated string

of  digits.    Leading  blanks  are

ignored.    A leading sign (+,-) is

allowed.

int       iret;       -- atoi returns the string in integer

representation.

long      lret;       -- atol returns the string  in  long

representation.

double    fret;       -- atof returns the string in double

floating representation.

int       atoi(), atol()

double    atof();

#Syntax#

iret   =  atoi(string);

lret   =  atol(string);

fret   =  atof(string);

#Notes#

Checking for overflow condition is not handled by atoi, atof

or atol.

.pa

##brk,_sbrk##

The brk and sbrk functions set the top of the program heap.   brk

sets  the upper bound (above the current upper bound and referred

to  as  the break) of the program heap to  a  specified  address.

sbrk increases the heap by a specified number of bytes.

#Declarations#

char      *addr;      -- the new break address

int       incr;       -- amount to  increase  heap by  using

sbrk.

int       bret;       -- 0 if brk is successful, -1 if not.

char      *bheap      -- pointer   to   beginning   of  heap

extension,    -1    if   sbrk    is

unsuccessful.

int       brk();

char      *sbrk;

#Syntax#

bret = brk(addr);

bheap = sbrk(incr);

.pa

##calloc,_malloc,_realloc,_free##

The  calloc,  malloc,  realloc  and  free functions are  used  to

manipulate the allocation and deallocation of blocks (spaces)  of

memory  in  the  heap.   calloc (chunk  allocation)  returns  the

starting  address  of  a space allocated for  an  array.   malloc

(memory  allocation)  returns the starting address  for  a  space

allocated  on a word boundary.    realloc (re-allocation) returns

the starting address of a space that has been changed in size  by

the  specified amount.   free returns the specified space to  the

heap.

#Declarations#

int       size        -- the number of bytes to allocate for

the space.

int       asize;      --  the  size  in bytes of  one  array

element.

int       narray;     -- the  number of array  elements  to

allocate for the space.

char      *sret       -- pointer to the allocated space.

char      *addr       -- pointer  to the beginning  of  an

existing space.

char      *calloc(); *malloc(); *realloc();

#Syntax#

sret   =  calloc(narray, asize);

sret   =  malloc(size);

sret   =  realloc(addr, size);

free(addr);

#Notes#

When  realloc  is called,  the information in  the  original

space is copied (if necessary) to the newly allocated space.   If

there  is sufficient adjacent space,  realloc uses it as part  of

the original space,  avoiding the costly copy operation.  Realloc

frees the old space if it is not part of the new space.

.pa

##close##

The close function closes an open (opened via open or creat) file

or  device.   Note,  you  must specify a file descriptor for  the

close function, not a stream address.  To close a stream file use

the fclose function.

#Declarations#

int       fd;         -- file  descriptor of a  valid  open

file.

int       ret;        -- 0 if close succeeds, -1 if it fails

int       close();

#Syntax#

ret   =   close(fd);

.pa

##cos,_sin,_tan,_atan,_exp,_log,_log10,_sqrt##

The math functions consist of cos,  sin,  tan,  atan,  exp,  log,

log10   and  sqrt.    Each  function  returns  their   respective

mathematical values.

#Declarations#

double    num;        -- a  double precision floating  point

number  that expresses the angle in

radians.

double    fret;       -- the  returned value of one  of  the

following functions.

double    sin():      -- trigonometric  sine   of  an  angle

expressed in radians.

double    cos();      -- trigonometric  cosine  of an  angle

expressed in radians.

double    tan();      -- trigonometric  tangent of an  angle

expressed in radians.

double    atan();     -- trigonometric   arc  tangent  of  a

number.

double    exp();      -- the  value of the constant e raised

to   the   specified  argument   (e

= 2.7182812845905)

double    log();      -- the  logarithm  base e of a  number

(natural logarithm).

double    log10();    -- the logarithm base 10 of a number.

double    sqrt();     -- the square root of a number.

#Syntax#

fret   =  sin(num);

fret   =  cos(num);

fret   =  tan(num);

fret   =  atan(num);

fret   =  exp(num);

fret   =  log(num);

fret   =  log10(num);

fret   =  sqrt(num);

.pa

##creat,_creata,_creatb##

The creat,  creata and creatb functions create new files.   creat

and  creata  both create ASCII files.   creatb creates  a  binary

file.  Binary  files  are generally faster to access  than  ASCII

files  because of computational overheads in handling the CR  and

LF characters.

#Declarations#

char      *filename;  -- filename of file to create.

int       mode;       -- access mode, not used by the DSI-32

interface (for UNIX compatibility).

int       fd;         -- file  descriptor returned by creat,

creata and creatb.

int       creat(); creata(); creatb();

#Syntax#

fd    =   creat(filename, mode);

fd    =   creata(filename, mode);

fd    =   creatb(filename, mode);

#Notes#

creat  and  creata are functionally  equivalent.  Additional

information  on  the creat,  creata and creatb functions  may  be

found in #The_C_Programming_Language# by Kernighan and Ritchie.

.pa

##isalpha,_isupper,_islower,_isdigit,_isalnum,_isspace,_ispunct,

isprint,_iscntrl,_isascii##

These  functions return true or false if the condition  specified

by their name is or is not met, respectively.

#Declarations#

#include  <ctype.h>

char      c;          -- character to be checked.

int       cret;       -- 0  if condition is met,  -1 if  it

fails.

#Syntax#

cret   =  isalpha(c); -- is c an alpha character (a-z, A-Z)

cret   =  isupper(c); -- is c uppercase (A-Z)

cret   =  islower(c); -- is c lowercase (a-z)

cret   =  isdigit(c); -- is c a digit (0-9)

cret   =  isalnum(c); -- is c alphanumeric (a-z, A-Z, 0-9)

cret   =  isspace(c); -- is c a white space character  (SP,

TAB, CR, LF, FF)

cret   =  ispunct(c); -- is  c  a  punctuation  character

(.,;,!, etc)

cret   =  isprint(c); -- is c a printable character

cret   =  iscntrl(c); -- is c a control character

cret   =  isascii(c); -- is c an ASCII character (between 0

and 128)

#Notes#

These are macros defined in CTYPE.H and therefore,  must not

be declared in the user source.

.pa

##exit##

The exit function returns control to the operating system.   exit

flushes  all  buffers,   closes  all  open  files  and  sets  the

completion code.

#Declarations#

int        code;    -- completion  code returned to  the

operating system.

#Syntax#

exit(code);

#Notes#

exit  does not return a value and therefore does not need to

be declared.

.pa

##fclose,_fflush##

The fflush function writes out to disk any data in a stream file.

fclose does the same, but closes the file.

#Declarations#

FILE      *stream;  -- pointer to a stream file.

int       fret;     -- 0 if flush succeeds, -1 if it fails.

int       fclose(), fflush();

#Syntax#

fret   =  fclose(stream);

fret   =  fflush(stream);

#Notes#

Additional  information  on  streams is available  in  #The_C

Programming_Language# by Kernighan and Ritchie.

.pa

##ferror,_clearerr,_feof,_fileno##

The  feof,  ferror,  clearerr  and fileno are used to  manipulate

stream  files.   They  are macros  defined  in  STDIO.H.   ferror

returns an error status on a stream.   clearerr clears any errors

on the specified stream.   feof determines whether an end of file

has  been  reached  on a stream or not.   fileno returns  a  file

descriptor associated with the specified stream.

#Declarations#

FILE      *stream;  -- stream to operate on.

int       fret;     -- Non zero if at end of file, 0 if not.

int       eret;     -- Non zero if error, 0 if no error.

int       fd;       -- file  descriptor  associated  with  a

stream.

#Syntax#

fret   =  feof(stream);

eret   =  ferror(stream);

fd     =  fileno(stream);

clearerr(stream);

#Notes#

As with all other macros they must not be defined to  return

anything.   Additional information on streams is available in #The

C_Programming_Language# by Kernighan and Ritchie.  Also see fopen.

.pa

##fopen,_fopena,_fopenb##

The fopen, fopena, fopenb functions create an association between

a  file or device and a stream.   fopen and fopena both associate

streams  with  ASCII files.   fopenb associates a stream  with  a

binary  file.  Note  that  binary files are generally  faster  to

access  than  ASCII  files  due  to  computational  overheads  in

processing the CR and LF characters.

#Declarations#

char      *filename; -- pointer  to  a  null  terminated

filename.

char      *access;   -- a  character  with  one  of  the

following values:

r - read access

w - write access

a - append access

int       fd;        -- file descriptor associated with an

existing open file.

FILE      *stream;

FILE      *fopen(), *fopena(), *fopenb();

#Syntax#

stream  = fopen(filename, access);

stream  = fopena(filename, access);

stream  = fopenb(filename, access);

.pa

##fread,_fwrite##

The fread and fwrite functions are used to move byte data between

a  stream  and  memory.   fread reads from a  stream  to  memory.

fwrite writes from memory to a stream.

#Declarations#

char      *buffer;  -- pointer to a memory buffer.

int       size;     -- size  in bytes of an item  in  the

stream.

int       rnitems;  -- requested number of items read  or

written in the stream.

int       nitems;   -- actual  number of items  read  or

written in the stream.

FILE      *stream;  -- stream to read from and write to.

int       fread(), fwrite();

#Syntax#

nitems  = fread(buffer, size, rnitems, stream);

nitems  = fwrite(buffer, size, rnitems, stream);

#Notes#

The  requested  number of items read or written may  not  be

equal to the actual number of items read or written.   A value of

0 is returned if an error or EOF occurs.

.pa

##fseek,_ftell,_rewind##

The  fseek,  ftell and rewind functions allow positioning of  the

read/write  pointer in a stream.   fseek sets the pointer to  the

specified  location.   ftell returns the current position of  the

pointer  in  the  stream.   rewind  places  the  pointer  at  the

beginning of the stream.

#Declarations#

long      offset;   -- number of bytes to move pointer from

current position (signed value).

int       offtype;  -- type of offset:

0 - offset from start of file

1 - offset from current location

2 - offset from end of file

FILE      *stream;  -- stream to position

int       fret;     -- 0 if positioning succeeds,  -1 if it

fails.

int       fseek(), rewind();

long      ftell();

#Syntax#

fret   =  fseek(stream, offset, offtype);

offset =  ftell(stream);

fret   =  rewind(stream);

.pa

##getc,_getchar,_fgetc,_getw,_getl##

getc  and  getchar are macros that  read  characters.   They  are

defined in STDIO.H.  fgetc, getw and getl are functions that read

characters,  words and long integers from a stream.  getc reads a

character from a stream.  getw reads a 16-bit word from a stream.

getl reads a 32-bit long integer from a stream.   getchar reads a

character  from  stdin  (the  standard  input).   fgetc  reads  a

character  from  a stream.   getc is a macro and is  functionally

equivalent to fgetc, which is a function.

#Declarations#

FILE      *stream;  -- stream to read from.

int       cret;     -- character returned from getc, fgetc

and getchar.

int       wret;     -- word returned from getw.

long      lret;     -- 32-bit long integer  returned  from

getl.

int       fgetc(), getw();

long      getl;

#Syntax#

cret   =  getc(stream);

cret   =  fgetc(stream);

cret   =  getchar();

wret   =  getw(stream);

lret   =  getl(stream);

#Notes#

Errors  returned  by the stream read functions may be  valid

values.  Use feof and ferror to detect errors when using streams.

Also  as in all other case of macros,  do not declare the  macros

getc() and getchar().

#getpid#

The  getpid  function  is included for compatibility  with  UNIX.

Under  UNIX,  getpid  returns  the  process  id.  In  the  DSI-32

implementation a random number is returned.

#Declarations#

int       pid, getpid();

#Syntax#

pid   =   getpid();

.pa

##gets,_fgets##

The  gets and fgets functions read strings from a  stream.   gets

reads  a  string from stdin (the standard  input),  removing  the

newline character at the end of the string.  fgets reads a string

from  a stream and leaves the newline character in  the  returned

string.

#Declarations#

char      *cstring; -- pointer to a null terminated string.

int       maxc;     -- maximum character count for fgets.

FILE      *stream;  -- stream to read from.

char      *rstring  -- return string (= cstring) if gets or fgets

succeeds, null if fails.

char      *gets(), *fgets();

#Syntax#

rstring = gets(cstring);

rstring = fgets(cstring, maxc, stream);

#Notes#

The  string cstring holds the input string in both gets  and

fgets.   Adequate space for an input line must be provided for by

the  string  cstring.   Maxc  specifies  the  maximum  number  of

characters  to  be  returned  in  fgets,  including  the  newline

character.

.pa

##index,_rindex,_strchr,_strrchr##

The  index,  rindex,  strchr  and  strrchr functions  return  the

location of a specified character in a string.   index and rindex

search for the first and last occurrence of a substring within  a

string respectively.  strchr and strrchr search for the first and

last occurrence of a character within a string respectively.

#Declarations#

char      c;          -- character to locate in string.

char      *sbstring;  -- pointer to substring to search for.

char      *cstring;   -- pointer to null terminated  string

to search.

int       rchar;      -- offset corresponding to position of

substring  or character with string

or -1 if no match.

char      *index(), *rindex(), *strchr(), strrchr();

#Syntax#

rchar  =  index(cstring, sbstring);

rchar  =  strchr(cstring, c);

rchar  =  rindex(cstring, sbstring);

rchar  =  strrchr(cstring, c);

.pa

##lseek,_tell##

lseek  and  tell are similar to fseek and ftell except that  they

function on file descriptors instead of streams.

#Declarations#

long      offset;    -- number of bytes to move pointer from

current position (signed value).

int       offtype;   -- type of offset:

0 - offset from start of file

1 - offset from current location

2 - offset from end of file

int       fd;        -- file to position

long      fret;      -- 0 if positioning succeeds, -1 if it

fails.

long      lseek(), tell();

#Syntax#

fret   =  lseek(fd, offset, offtype);

offset =  tell(fd);

.pa

##mktemp##

The  mktemp  function makes a temporary filename.   Use  this  in

conjunction  with  creat,  creata  or creatb to create  a  unique

temporary file.   The template is a character string containing a

user  defined prefix followed by six x characters (upper case  or

lower  case).   The  x's  are replaced by  a  unique  number  and

returned in the original template character string.

#Declarations#

char      *fstring;   -- pointer  to  a  null   terminated

filename template.

char      *mktemp;

#Syntax#

fstring = mktemp(fstring);

.pa

##open,_opena,_openb##

The open and opena functions open existing ASCII files and return

a  file  descriptor for the opened file.   openb  opens  existing

binary files and return a file descriptor for the opened file.

#Declarations#

char      *filename;  -- pointer  to a null  terminated

filename.

int       mode;       -- access mode which is one  of  the

following:

0 - read access

1 - write access

2 - read/write access

int       fd;         -- file descriptor returned by open,

opena or openb -1 if it fails.

int       open(), opena(), openb();

#Syntax#

fd    =   open(filename, mode);

fd    =   opena(filename, mode);

fd    =   openb(filename, mode);

.pa

##perror##

The  perror function writes an error message corresponding to the

last  system  error  to  stderr  (the  standard  error   output).

External  variable  errno contains the value of the  last  system

error  code which is used by perror to determine the last  system

error message.

#Declarations#

include   <errno.h>

char      *pstring; -- pointer to a null terminated prefix.

#Syntax#

perror(pstring);

#Notes#

pstring  is  a string which is written prior  to  the  error

message.   See  ERRNO.H  for a description of the  defined  error

codes and messages.

.pa

##printf,_fprintf,_sprintf##

The  printf,  fprintf and sprintf interpret a format  string  and

arguments to output to the standard output,  a stream and memory,

respectively.   The  number  of  arguments must be  equal  to  or

greater  to the number of format operators in the format  string,

otherwise  undefined  results may occur.   Format  operators  are

represented  as  a  %  (percent)  sign,   followed  by   optional

formatting  symbols,  followed  by  a single  character  operator

designator.    A  space  or  equivalent  white  space   character

separates character operator designators.

Formatting symbols consist of the following:

-          - (minus sign) left justify conversion in field

w.p        - (w,   p  =  decimal  digit)  field  width   and

precision. precision is optional

L or l     - specifies a 32-bit value

Format operator designators consist of the following:

d    - convert binary argument to decimal

o    - convert binary argument to octal

x    - convert binary argument to hexadecimal

X    - convert binary argument to uppercase hexadecimal

u    - convert binary argument to unsigned decimal

s    - argument is a string

c    - argument is a single character

e    - convert floating argument to scientific notation

f    - convert  floating  argument  to  standard   numeric

notation

g    - convert  floating  argument  to  %e  of  %f  format,

whichever is shortest

.pa

#Declarations#

char      *format;    -- pointer to a null terminated format

descriptor string.

char      *dstring;   -- pointer  to  a  null   terminated

destination string for sprintf.

FILE      *stream;    -- destination stream for fprintf.

int       nret;       -- number of characters output.

int       arg1, arg2,..., argN;

-- N  arguments of type  corresponding

to  respective  character  operator

designator.

#Syntax#

nret   =  printf(format, arg1, arg2,..., argN);

nret   =  fprintf(stream, format, arg1, arg2,..., argN);

nret   =  sprintf(dstring, format, arg1, arg2,..., argN);

#Notes#

A complete discussion of printf, fprintf and sprintf appears

in #The_C_Programming_language# by Kernighan and Ritchie.

.pa

##putc,_putchar,_fputc,_putw,_putl##

putc  and  putchar  are  macros defined  in  STDIO.H  that  write

characters to a stream.  fputc,  putw and putl are functions that

write  characters,  words and long integers to  a  stream.   putc

writes  a character to a stream.   putw writes a 16-bit word to a

stream.   putl writes a 32-bit long integer to a stream.  putchar

writes a character to stdout (the standard output).  fputc writes

a  character to a stream.   putc is a macro and  is  functionally

identical to fputc, which is a function.

#Declarations#

char      c;          -- character to write to stream.

int       w;          -- word to write to stream.

long      l;          -- 32-bit  long  integer to  write  to

stream.

FILE      *stream;    -- stream to write to.

int       cret;       -- character written by putc, fputc or

putchar, -1 if an error occurs.

int       wret;       -- word  written by putw,  -1  if  an

error occurs.

long      lret;       -- 32-bit  long integer  written  by

putl, -1 if an error occurs.

int       putchar(), fputc(), putw();

long      putl;

#Syntax#

cret   =  putc(c, stream);

cret   =  fputc(c, stream);

cret   =  putchar(c);

wret   =  putw(w, stream);

lret   =  putl(l, stream);

.pa

#Notes#

Errors  returned by the stream write functions may be  valid

values.  Use feof and ferror to detect errors when using streams.

.pa

##puts,_fputs##

The  puts  and fputs functions write strings out to  stdout  (the

standard  output)  and a specified output  stream,  respectively.

puts appends a newline character at the end of the output  string

whereas fputs does not.

#Declarations#

char     *cstring;  -- pointer to null terminated  character

string.

FILE      *stream;

int       sret;     -- last  character output,  or -1  if

output fails.

int       puts(), fputs();

#Syntax#

sret   =  puts(cstring);

sret   =  fputs(stream, cstring);

.pa

##rand,_srand##

The rand and srand functions return pseudo-random numbers.  srand

initializes  the random number generator with the  user  supplied

seed value and returns a random number.   rand returns subsequent

random numbers.

#Declarations#

int       seed;      -- initial seed value chosen by user.

int       rret;      -- random value returned by generator.

int       rand(), srand();

#Syntax#

rret   =  srand(seed);

rret   =  rand(seed);

#Note#

The rand function returns and pseudo random number between 0

and 2029052025-1.

.pa

##read##

The  read  function reads input from an open file specified by  a

file descriptor.

#Declarations#

int       fd;        -- the  file  descriptor  defining  the

file to read from.

char      *mbuffer;  -- pointer to memory buffer to  receive

the input data.

unsigned  n;         -- number of bytes to read.

int       rret;      -- number of bytes read,  -1 if an error

occurs.

int       read();

#Syntax#

rret   =  read(fd, mbuffer, n);

.pa

##scanf,_fscanf,_sscanf##

The scanf, fscanf and sscanf functions read input from stdin (the

standard   input),   a  stream  and  a  null  terminated  string,

respectively.    The  input  is  interpreted  according  to   the

specified  format descriptor string and placed into the remaining

arguments to the function.

The  arguments may be of any type corresponding to the conversion

specifications  in the format descriptor string.   The number  of

conversion specifications in the format descriptor string must be

less  than  or equal to the number of arguments that  follow  the

descriptor.

A conversion specification consists of a percent  sign,  followed

by  an  optional  * (asterisk),  followed by  a  format  operator

designator.   The * specifies that the data is not to be assigned

to the argument.   Format specification designators are separated

by a space or equivalent white space character.

Format operator designators consist of the following:

h  -  convert short ASCII integer string to binary

d  -  convert decimal ASCII integer string to binary

o  -  convert octal ASCII integer string to binary

x  -  convert hexadecimal ASCII integer string to binary

s  -  output a (space terminated) string

c  -  output a character

e  -  convert scientific notation ASCII string to binary

f  -  convert  standard  numeric notation ASCII  string  to

binary

.pa

#Declarations#

char      *format;   -- pointer to a null terminated  format

descriptor string.

char      *sstring;  -- pointer to a null terminated source

string for sscanf.

FILE      *stream;   -- source stream for fscanf.

int       nret;      -- number of conversion specifications

converted.

int       *arg1,*arg2,...,*argN;

-- pointers  to  arguments of any  type

corresponding  to format descriptors

string.

int       scanf(), fscanf(), sscanf();

#Syntax#

nret   =  scanf(format, arg1, arg2,..., argN);

nret   =  fscanf(stream, format, arg1, arg2,..., argN);

nret   =  sscanf(sstring, format, arg1, arg2,..., argN);

#Notes#

Additional  information on scanf,  fscanf and sscanf can  be

found in #The_C_Programming_Language# by Kernighan and Ritchie.

.pa

##setjump,_longjmp##

The setjump and longjmp functions are used in programs which  use

nonlocal  goto  statements.   setjump stores the  existing  local

program  environment prior to a nonlocal goto statement.  setjump

also stores the location at which execution is to continue  after

a longjmp call.   longjmp simulates a return from a nonlocal goto

statement to the location saved when setjump was called.  longjmp

returns a status value to setjump upon return.

#Declarations#

#include  <setjump.h>

jmp_buf   env;      -- environment saved by setjump.

int       sret;     -- returned  value  after  a  return  to

setjump environment.  Initially, this

value  is  0