AccessionIndex: TCD-SCSS-T.20251216.009
Accession Date: 16-Dec-2025
Accession By: Dr.Brian Coghlan and Erturk Kocalar
Object name: Arduino shield for Intel 8085
Vintage: 2025
Synopsis: A board that enables execution of software by the 8085, Intel’s second-generation 8-bit microprocessor.
Description:
This Retroshield 8085 is one a series of Arduino shields (retroshields) designed by Erturk Kocalar [1] to not only host early microprocessors but also to execute their original native software, an approach he has termed “Breadboarding in Software”. 

The RetroShield 8085 [2] is a daughter card that plugs into an Arduino Mega [3] or Teensy [4], see Fig.1. The Intel 8085 microprocessor [5] executes native 8085 code while the Arduino emulates 8085 system hardware. The choice of emulated 8085 system hardware is done by uploading relevant Arduino code. This enables the experience of old computer systems, the learning of low-level aspects of the early microprocessors, the keeping of valuable historical software alive, and even changing original native software simply by changing C code in the Arduino IDE, and use with other Arduino shields and libraries.
The Arduino Mega is an inexpensive platform with a 16MHz CPU that allows the retroshield’s early microprocessor to run at about up to 400kHz, with over 200kB of emulated ROM and about 6kB of emulated RAM (reserving 2kB for the Arduino stack/heap). Peripherals like a UART, PIA, Timer, etc. can be emulated by the Arduino, and other Arduino shields can be used to add new features. The Arduino Mega operates at 5V TTL levels. The retroshield plugs into the Arduino Mega digital input/output connector.
The Teensy is a more expensive but faster platform with a 600MHz CPU that allows the retroshield’s early microprocessor to run at full speed (e.g. 1MHz), with over 512kB of emulated ROM. Emulated RAM capacity depends on the Teensy version, 256kB for Teensy 3.5 or 3.6, 1MB for Teensy 4.1. It includes a microSD slot that can be used to emulate disk drives. Other peripherals like a UART, PIA, Timer, etc. can be emulated by the Arduino, and other Arduino shields can be used to add new features. The Teensy operates at 3.3V, but plugs into a Teensy daughter card designed by Erturk Kocalar that provides onboard 3.3V-to-5V TTL level shifting & vice-versa. The retroshield then plugs into that daughter card in the same way that it would into an Arduino Mega digital input/output connector.
The 8085 replaced Intel’s 8080, which despite being fabricated in nMOS, required +5V, −5V, and +12 V power supplies. The 8085 was again fabricated in nMOS, but only needed a single +5V power supply.

The 8085 had the same register set as the 8080, and was binary compatible with it, but had a slightly different architecture, including a serial communications interface, an internal clock generator, and two new instructions to ease interrupt handling. But apart from the power supplies, the major difference was that it freed up 8 pins by multiplexing the 8-bit data bus with the lower 8-bits of the address bus. Hence it was able to use those 8 pins plus the 2 pins that had served -5V and +12V for 10 new signals: ALE, TRAP, RST7.5, RST6.5, RST5.5, SID, SOD, S0, S1, IO/M. The 8085 also replaced the signals INTE, DBIN, SYNC, WAIT with /INTA, RESOUT, /RD, HLDA. In general it was simpler to design with and much faster. The Retroshield 8085 design files are at [6].
The Retroshield 8085 can execute the native Intellec 8/Mod 8 ROM-resident monitor [7][8] that supports memory, a PROM programmer, a tape/punch, and a Teleytype. An Editor/Assembler for this appears to have been lost. It can also execute native SCELBAL (SCientific ELementary BAsic Language) [9], which is documented in book form [10] and has floating point mathematical functions (sin, cos, exp, log, atn, and strings and arrays). The Retroshield 8085 software is at [11].
Many thanks to Erturk Kocalar and Brian Coghlan for donating this item.

The homepage for this catalog is at: https://www.scss.tcd.ie/SCSSTreasuresCatalog/
Click 'Accession Index' (1st column listed) for related folder, or 'About' for further guidance.

Some of the items below may be more properly part of other categories of this catalog, but are listed here for convenience.

	Accession Index
	Object with Identification

	TCD-SCSS-T.20251216.009



	Arduino shield for Intel 8085. A board that enables execution of software by the 8085, Intel’s second and very successful early 8-bit microprocessor, 2025.

	TCD-SCSS-T.20250918.001


	Intel 8080 microprocessor and associated chips. Intel’s second and very successful early 8-bit microprocessor. 1974.

	TCD-SCSS-T.20251216.001



	Arduino shield for Intel 8080. A board that enables execution of software by the 8080, Intel’s second and very successful early 8-bit microprocessor, 2025.

	TCD-SCSS-X.20250916.001


	Dr.Brian Coghlan's Collection of Early Microprocessors. An extensive and nearly complete set of unused 1970s microprocessor chips, most accompanied with documentation, some with demonstration boards. 1971.

	
	


References:
1. Erturk Kocalar, 8bitforce, erturkk@8bitforce.com, see:
https://8bitforce.com/
Also: https://gitlab.com/8bitforce
Last browsed to on 18-Sep-2025.

2. Erturk Kocalar, Retroshield 8085, see:

https://www.tindie.com/products/8bitforce/retroshield-8085-for-arduino-mega/
Last browsed to on 18-Sep-2025.

3. Arduino, Arduino MEGA, see:

https://store-usa.arduino.cc/products/arduino-mega-2560-rev3
Last browsed to on 18-Sep-2025.

4. PJRC, Arduino Teensy, see:

https://www.pjrc.com/teensy/
Last browsed to on 18-Sep-2025.

5. Wikipedia, Intel 8085, see:

https://en.wikipedia.org/wiki/Intel_8085
Last browsed to on 18-Sep-2025.

6. Erturk Kocalar, Retroshield-HW Design Files, see:

https://gitlab.com/8bitforce/retroshield-hw
Last browsed to on 18-Sep-2025.

7. Wikipedia, Intellec, see:

https://en.wikipedia.org/wiki/Intellec
Last browsed to on 18-Sep-2025.

8. Herb Johnson, Intel MON-80 PROMs and systems, 2024, see:

https://www.retrotechnology.com/restore/mon80_proms.html
Last browsed to on 18-Sep-2025.

9. SCELBI Computer Museum, SCELBI, see:

https://www.scelbi.com/
Last browsed to on 18-Sep-2025.

10. Mark Arnold and Nat Wadsworth, SCELBAL: A Higher Level Language for 8008/8080 Systems, pp.372, Scelbi Computer Consulting, 1976, see:

https://www.scelbi.com/
Last browsed to on 18-Sep-2025.

11. Erturk Kocalar, Retroshield 8008 Source Code, see:

https://gitlab.com/8bitforce/retroshield-teensy/-/tree/master/t8085
Last browsed to on 18-Sep-2025.

[image: image1.jpg]



Figure 1: Intel 8085 chip die micrograph (from Wikipedia).
[image: image2.jpg][ RST65 AP S son Intel 8085 Microarchitecture.

wge fespss [ TR t
4 3 4
: Serial 10
- Contrl
& Bitinternal Data Bus.
Accumutzor | [ Temp.
o de nstructon
Register
e | @80
Tre | ore
e |
1 =5
we | s
Instruction 5 [HRes | Lreg a8t
Decoder and 2| een | wa
il =
o (16 5i)
cycle e Coe
Encoding Gaen
e
yrivy
Addss 4 o,
Jram—— [
[
Wutilexer
= S I
el ReseT :ml lmlms conTRoL Address Buter | | Dataiadaress Bufer
cuxour ] REsErour] HoA SO st

RESETIN# HoLD

1ADO-AD7

A7 DODT
Address Bus Data Bus





Figure 2: Intel 8085 architecture (from Wikipedia).
[image: image3.jpg]Intel 8085 registers

A414.1,1.0.0.0.0. 0.0, 0,00 0 minas
543210098765 4 3271 o /biposion)

M - Program Status Word
B

D

H (indirect address)

g

Index registers

Stack Pointer
Program counter
Program Counter

EEREEERE -

Interrupt Mask Register

1 it s
o o s v e

73
:
3
‘%
2





Figure 3: Intel 8085 registers (from Wikipedia).
[image: image4.jpg]_.4
=
=]

2 39
3 38
4 37
5 36
6 35
7 34
8 33
9 32
10 3
" 8085 5
12 29
13 28
27
15 26
16 25
17 24
18 23
19 22
0 21





Figure 4: Intel 8085 pinout (from Wikipedia).
[image: image5.jpg](LOWORDER  paTA FROM DATA FROM MEMORY 'DATA TO MEMORY
ADDRESS) MEMORY. (/0 PORT ADDRESS) OR PERIPHERAL
(INSTRUCTION)
aLe , \ ' \ , \ , \
b
wa

status.

23171814





Figure 5: Intel 8085 instruction cycle (from Intel datasheet).
[image: image6.jpg],_.O. ( OOCOQQQQQQQQQQQQ_

- ¥l |

Q!!!!!!Diiiiiiiiiliﬁ

N ,w " 607100 .~
| ...zmzmoomg?m :
@MITSUBISHI

,@@@m-aaa@.mmma&asaﬁ

E:::::__

Q Q «::;IC.,O.

J g o . : ¢ g
*l . . L8 , “l




  [image: image7.jpg]i (0

G . % ¥ b L’
T S N O O O SO VNS \' - ST - Y- Y7 N0 v

da@aaecaoacoqéiiiéti

,.G\“‘i“@qﬁ‘\‘j".1a‘ Al Nl Nl N

. « € {
i
A st an T an o Y ek Y dh P N o g

.,4‘1‘!‘ !d“.‘l}’.‘l“l‘lv
. ...‘

N
AU

Q@QQQQQQC

{ _ \...V% ./2
| « v )

_v_.w.xmezoz m:_n_ ﬁm.r_ 1
h&m L Oﬂ@?_ QS.JWLG 50&@6&&

LSPSUL0DLOZN

mﬁftlllklii}Fffff[f,





Figure 6: 8bitforce Retroshield 8085 (from 8bitforce).
[image: image8.jpg]- b T, 0

000

R, /*..
Ny

‘ gd
_m,‘..m m

T

1=

?:’:4:2:-

HH

{11:

R

3

= W
= 7 (73}
W0J'00Fa[e' MMM i

R -~

iy B
3





Figure 7: 8bitforce Retroshield 8085 coupled with Arduino Mega.

[image: image9]
Figure 8: 8bitforce Retroshield 8085 assembly instructions.

[image: image10]
Figure 9: 8bitforce Retroshield 8085 schematic.
[image: image11.jpg]HYIYOOMN MHN1YT A8 NMvHa

1NO 1333
NI_ 13834

a3y .
La3q

S080 089
¢y

aIeMmpJe;
82.nos uado

il

z

L 9gH3avay
8LXeHAHVOIW
% e



[image: image12.jpg]Hne .t_u._.mn_‘____.g. mu,.‘cw.._h,onfouo_)_m,ou ssaJ0e U\n_ INJaJed aq usyj ‘siapeay cumﬁm._o“ u\cm>>:.o>t |
- S . 4 S sjuiod aqouid

4UQ0T = T2
:1apJo s1y1 ul syed 13pjos .
: 'S808

wo2°9210431g8 ‘6T0C Y3AdoD
80/60/6T0T ‘V A3y
suonanasu A|quiassy S808 PI21YSo119y




